@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

# REVISION  TRANSMITTAL  SHEET

AIRBUS

CUSTOMER  SERVICES  DIRECTORATE

1  Rond  Point  Maurice  Bellonte

31707  BLAGNAC  CEDEX  FRANCE

Tel       :  (33)  5  61  93  33  33

Telex  :  AIRBU  530526F

Fax       :  (33)  5  61  93  42  51

ATA  SYSTEM  :  21

TITLE  :  AIR  CONDITIONING  -  COCKPIT  AIR  HEATING  -  IMPROVE  FOOT  WARMER  EFFICIENCY .

MODIFICATION  No .  :  35861P9435  35861P9449

This  page  transmits  Revision  No .  01  of  Service  Bulletin  No .  A320-21-1908 . ADDITIONAL  WORK

No  additional  work  is  required  by  this  revision  for  aircraft  modified  by  any

previous  issue .

REASON

Revision  No .  01  issued  to  add  aircraft  MSN  2233  and  MSN  2243,  to  include    design  office  changes  and  to  include  modifications  after  the  validation  on A321-211  aircraft  MSN  2342.

CHANGES

SUMMARY  :

—  REASON/DESCRIPTION/OPERATIONAL  CONSEQUENCES

.    Kit  211908A05R01  price  added  in  evaluation  table .

—  EFFECTIVITY

.    Sentence  added .

—  MANPOWER

.    Config .  01  :  3.0  TOTAL  MANHOURS  become  10.0  and  1.5  ELAPSED  TIME  hours

become  6.0.  Config .  02  :  108.0  TOTAL  MANHOURS  become  109.0  and  57.0

ELAPSED  TIME  hours  become  58.0.  Config .  03  :  manpower  added .

—  MATERIAL  INFORMATION

.    Material  information  added  for  kit  211908A05R01 .

PLANNING  INFORMATION  :

—  EFFECTIVITY

.  Para .  1 .A .  Aircraft  MSN  2233  and  MSN  2243  added  for  operator  AFL .  Note

Config .  03  added .

5    DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  1  of  4

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

REVISION  TRANSMITTAL  SHEET

—  DESCRIPTION

.  Para .  1 .D .  Config .  01  modified .  Config .  03  added .

—  MANPOWER

.  Para .  1 .G .  Config .  01  :  3.0  TOTAL  MANHOURS  become  10.0  and  1.5  ELAPSED

TIME  hours  become  6.0.  1.0  added  for  tests .  Config .  02  :  108.0  TOTAL

MANHOURS  become  109.0  and  57.0  ELAPSED  TIME  hours  become  58.0.  6.0  for

test  become  7.0.  Config .  03  :  manpower  added .

—  WEIGHT  AND  BALANCE

.  Para .  1 .H .  Paragraph  updated  for  config .  03.

—  ELECTRICAL  LOAD  DATA

.  Para .  1 .I .  Paragraph  updated  for  config .  03.

—  REFERENCES

.  Para .  1 .J .  Reference  to  AMM  23-71-00  added .

—  INTERCHANGEABILITY/MIXABILITY

.  Para .  1 .L .  PN  and  note  added  for  config .  03.

MATERIAL  INFORMATION  :

—  MATERIAL  -  PRICE  AND  AVAILABILITY

.  Para .  2 .A .  Price  added  for  kit  211908A05R01 .

—  LIST  OF  COMPONENTS

.  Para .  2 .C .  In  kit  211908A02R08  :  PN  D9550003000196  becomes

D9550003000197,  PN  D9000095220396  becomes  D9000095220397,  PN

D11311898A00  becomes  D11311898B00,  D33110306A00  becomes  D33110306C00,  PN

ASNA2080G01  becomes  ASNA2080G04,  PN  NSA931320-050A  becomes

NSA931320-050 .  Kit  211908A05R01  added .

—  LIST  OF  MATERIALS  -  OPERATOR  SUPPLIED

.  Para .  2 .D .  Rivets  PN  ASNA2080G01  added .

ACCOMPLISHMENT  INSTRUCTIONS  :

—  GENERAL

.  Para .  3 .A .  Config .  03  and  Config .  01  added  in  paragraph  Preparation .

—  MODIFICATION

.  Para .  3 .B .  Installation  of  the  panels  assy  added  in  Config .  01.  Text

added  or  modified  in  Config .  02.  Config .  03  added .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   2

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

REVISION  TRANSMITTAL  SHEET

—  TESTS

.  Para .  3 .C .  Tests  added  for  Config .  01  and  for  Config .  03.

—  CLOSE-UP

.  Para .  3 .D .  Config .  01  Tests  added .

—  RESTORATION  OF  AIRCRAFT  TO  ITS  INITIAL  CONFIGURATION

.  Para .  3 .G .  Config .  01  :  preparation  added .  Config .  03  added  for

restoration .

—  TESTS

.  Para .  3 .H .  Test  added  for  config .  01  and  Config .  03.

—  CLOSE-UP

.  Para .  3 .I .  Config .  01  :  close-up  added .  Config .  03  added .

ILLUSTRATIONS  :

| — Figure | 1 added . |
| --- | --- |
| — Figure | 4 added . |
| — Figure | 6 sheet 3 updated and sheet 4 added . |
| — Figure | 8, sheet 1 modified . |
| — Figure | 9, sheet 1 modified . |
| — Figure | 11, sheet 1 modified . |
| — Figure | 14, sheet 1 and sheet 2 added . |
| — Figure | 18, sheet 1 modified . |
| — Figure | 21, sheet 1 modified . |
| — Figure | 23, sheet 1 modified . |
| — Figure | 24, sheet 1 modified . |
| — Figure | 25, sheet 1 modified . |
| — Figure | 26, sheet 1 modified . |

FILING  INSTRUCTIONS

This  Service  Bulletin  has  been  generated  electronically  and  is  reissued  as  a

complete  document .  Replace  the  complete  document .

Put  this  Revision  Transmittal  Sheet  in  front  of  the  Service  Bulletin .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   3

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

REVISION  TRANSMITTAL  SHEET

HISTORY  OF  PREVIOUS  REVISIONS

No  previous  revisions .

REVISION  SEQUENCE

ORIGINAL  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   4

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

# SUMMARY

AIRBUS

CUSTOMER  SERVICES  DIRECTORATE

1  Rond  Point  Maurice  Bellonte

31707  BLAGNAC  CEDEX  FRANCE

Tel       :  (33)  5  61  93  33  33

Telex  :  AIRBU  530526F

Fax       :  (33)  5  61  93  42  51

This  summary  is  for  information  only

and  is  not  approved  for  modification  of  the  aircraft

ATA  SYSTEM  :  21

TITLE  :  AIR  CONDITIONING  -  COCKPIT  AIR  HEATING  -  IMPROVE  FOOT  WARMER  EFFICIENCY .

MODIFICATION  No .  :  35861P9435  35861P9449

REASON/DESCRIPTION/OPERATIONAL  CONSEQUENCES

On  longer  flight  on A320  family  aircraft,  low  temperatures  are  being  experienced

at  pilot  foot  level .  The  temperature  drop  is  proportional  to  the  length  of  the

flight,  and  pilot  reports  indicate  that  it  becomes  uncomfortable  after

approximately  3/3 . 5  hours  of  flight .  The  temperature  drop  appears  to  be  most

noticeable  on  newer  aircraft,  and  the  majority  of  pilot  reports  refer  to  "moving

air" .

Several  flight  tests  have  been  conducted  and  the  cold  feet  sensation  reported  by

the  airlines  has  been  reported  on  all  flights,  with  surface  and  air  temperatures

dropping  steadily  as  the  flight  length  increases .  The  cockpit  airflow  pattern

and  distribution  system  have  an  influence  on  the  low  temperatures  at  pilot  foot

level .  This  is  compounded  by  cooler  air  entering  the  cockpit  from  the  avionics

bay  and  by  temperature  losses  across  the  cockpit  distribution  ducting  from  one

air  outlet  to  another .  Cockpit  foot  warmers  improve  the  situation  as  per  their

basic  function .

To  improve  the  situation  a  package  of  modifications  have  been  developed  which

are  proposed  by  this  Service  Bulletin .

This  evaluation  Service  Bulletin  installs  :

—  On  aircraft  MSN  2342  only  :  Thermal

eliminate  cold  surfaces  which  could

convection .

insulation  around  the  forward  pedestal  to

cool  the  air  in  the  pedestal  area  by

—  On  all  aircraft  :

A  new  duct  routing  which  will  drive  the  air  to  a  better  location  to  ensure  a

good  comfort  of  the  pilot's  feet .

—  A  heater  in  the  cockpit  distribution  ducting  just  before  the  foot  level  outlet

(last  outlet  in  the  circuit)  to  allow  the  increase  of  the  outlet  temperature

at  this  level .

5    DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  1  of  4

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

SUMMARY

Accomplishment  of  this  evaluation  Service  Bulletin  will  provide  information  of all  evolution  (the  cockpit  air  heating,  thermal  insulation,  foot  warmer)            efficiency  in  operational  conditions  for  a  six  months  period .

| EVALUATION TABLE |  |  |  |
| --- | --- | --- | --- |
| COMPLIANCE | Desirable | CANCELS INSPECTION SB | No |
| POTENTIAL AD | No | A/C OPERATION AFFECTED | No |
| RELIABILITY AFFECTED | No | PAX COMFORT AFFECTED | No |
| COST SAVING | No | ETOPS AFFECTED | No |
| STRUCTURAL LIFE EXTN | No | VENDOR SB INVOLVED | No |
| KIT PRICE (USD) A02R08 | 15,230 | KIT PRICE (USD) A03R03 | 6,010 |
| KIT PRICE (USD) A04R00 | 37,410 | KIT PRICE (USD) A05R01 | 15,170 |

EFFECTIVITY

This  Service  Bulletin  is

AFL

NOTE  :  This  modification

applicable  to  this  operator  :

is  applicable  by  Service  Bulletin  only .

NOTE  :  This  Service  Bulletin  has  been  accomplished  at  Revision  No .  00  on

aircraft  MSN  2342  to  validate  its  content .

CONCURRENT  REQUIREMENTS

This  Service  Bulletin  cannot  be  accomplished  if  the  following  Service  Bulletin

has  been  accomplished:

Service  Bulletin  No .  A320-21-1909  Mod .  No .  35861P9430

REFERENCES/REPERCUSSIONS

TFU

OEB

AOT

SIL

LIFE  LIMIT

:  21 .21 .00 .022

:  None

:  None

:  None

:  None

LINE  MAINTENANCE  AFFECTED  :  No

OTHERS                                         :  None

NATURE  OF  THE  WORK

AIRCRAFT    :  YES

EQUIPMENT  :  NO

HARD  :  NO

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   2

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

SUMMARY

SOFT  :  NO

OBRM  :  NO

MANPOWER

For  Config .  01

TOTAL  MANHOURS                                                                                               10 .0

ELAPSED  TIME  (HOURS)                                                                                     6 .0

For  Config .  02  thru  Config .  03

TOTAL  MANHOURS                                                                                             109 .0

ELAPSED  TIME  (HOURS)                                                                                   58 .0

For  Config .  01  ,  restoration  of  the  aircraft

TOTAL  MANHOURS                                                                                               10 .0

ELAPSED  TIME  (HOURS)                                                                                     6 .0

For  Config .  02  thru  Config .  03  ,  restoration  of  the  aircraft

TOTAL  MANHOURS                                                                                             109 .0

ELAPSED  TIME  (HOURS)                                                                                   58 .0 MATERIAL  INFORMATION

AIRCRAFT  DATA

Kit  211908A02R08

Ducts,  brackets,  bundle,  panel,  placard

Kit  211908A05R01

Ducts,  brackets,  bundle,  panel,  placard

Kit  211908A03R03

Lateral  panels

Kit  211908A04R00

Heaters

APPENDICES

None

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   3

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

SUMMARY

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   4

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

# AIRBUS

CUSTOMER  SERVICES  DIRECTORATE

1  Rond  Point  Maurice  Bellonte

31707  BLAGNAC  CEDEX  FRANCE

Tel       :  (33)  5  61  93  33  33

Telex  :  AIRBU  530526F

Fax       :  (33)  5  61  93  42  51

ATA  SYSTEM  :  21

TITLE  :  AIR  CONDITIONING  -  COCKPIT  AIR  HEATING  -  IMPROVE  FOOT  WARMER  EFFICIENCY .

MODIFICATION  No .  :  35861P9435  35861P9449

This  document  contains  AIRBUS  PROPRIETARY  INFORMATION  and  shall  at  all  times      remain  the  property  of  AIRBUS;  no  intellectual  property  right  or  licence  is        granted  by  AIRBUS  in  connection  with  any  information  contained  in  it .  It  is        delivered  on  the  express  condition  that  said  document  and  the  information            contained  in  it  shall  be  treated  as  confidential,  shall  not  be  used  for  any        purpose  other  than  that  for  which  it  is  hereby  delivered .  It  shall  not  be            disclosed  in  whole  or  in  part  to  third  parties  and  shall  not  be  duplicated  in    any  manner  (except  for  the  purposes  of  performing  the  tasks  described  hereunder and  provided  that  any  recipient  of  such  document  shall  comply  with  the                  conditions  herein),  without  AIRBUS'  prior  written  consent .

1.     PLANNING  INFORMATION

## A .     EFFECTIVITY

(1)    Models

319-111  320-214  321-211

(2)    Aircraft

(a)    Effectivity  by  MSN

This  Service  Bulletin  is  applicable  to  aircraft  MSN  :

2233  2243  2330  2337  2342

NOTE  :  This  Service  Bulletin  has  been  accomplished  at  Revision

No .  00  on  aircraft  MSN  2342  to  validate  its  content .

(b)    Effectivity  by  Operator

The Operator/MSN  relationship  is  provided  for  information only and  is  correct  at  the  time  of  issue  in  accordance  with  the      information  available  to  AIRBUS .  Any  future  changes  resulting from  transfer  of  an  aircraft  from  one  operator  to  another will not  be  reflected  in  this  list  unless  the  Service  Bulletin  is  revised  for  another  reason .

5    DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  1  of  134

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

OPERATOR        MSN

AFL

2233  2243  2330  2337  2342

(c)    Effectivity  by  MSN  and  Kit/Configuration

The  kits  and  configurations  applicable  to  these  MSNs  are  given

at  the  end  of  this  list:

2330  2337  2342

KIT  No .

211908A02R08

211908A03R03

211908A04R00

QTY  PER  A/C

1

1

1

CONFIGURATION

02

01

02

NOTE  (01)  Config .  01  concerns  the  modification  No .  35861P9435

which  replaces  the  lateral  panels  on  the  center

pedestal  (only  for  aircraft  MSN  2342) .

NOTE  (02)  Config .  02  concerns  the  modification  No .  35861P9449 which  installs  the  equipment  and  the  associated        wiring  on  the  foot  air  outlet  in  the  cockpit  and      concerns  aircraft  with  panel  30VU  equipped  with  FIN 5HB  PN  ABS0951C3LM007 .

The  kits  and  configurations  applicable  to  these  MSNs  are  given

at  the  end  of  this  list:

2233  2243

KIT  No .            QTY  PER  A/C  CONFIGURATION

211908A04R00

211908A05R01

1

1

03

03

NOTE  :  Config .  03  concerns  the  modification  No .  35861P9449

which  installs  the  equipment  and  the  associated  wiring

on  the  foot  air  outlet  in  the  cockpit  and  concerns

aircraft  with  panel  30VU  equipped  with  FIN  5HB  PN

058-003-00 .

(3)    Spares

Placard

Placard

Placard

D11311478A00

723-4071-03

723-4207-01

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   2

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

## B .     CONCURRENT  REQUIREMENTS

This  Service  Bulletin  cannot  be  accomplished  if  the  following  Service

Bulletin  has  been  accomplished:

Service  Bulletin  No .  A320-21-1909  Mod .  No .  35861P9430

## C .     REASON

(1)    History

On  longer  flight  on  A320  family  aircraft,  low  temperatures  are

being  experienced  at  pilot  foot  level .  The  temperature  drop  is

proportional  to  the  length  of  the  flight,  and  pilot  reports

indicate  that  it  becomes  uncomfortable  after  approximately  3/3 . 5

hours  of  flight .  The  temperature  drop  appears  to  be  most  noticeable

on  newer  aircraft,  and  the  majority  of  pilot  reports  refer  to

"moving  air" .

Several  flight  tests  have  been  conducted  and  the  cold  feet

sensation  reported  by  the  airlines  has  been  reported  on  all

flights, with  surface  and  air  temperatures  dropping  steadily  as  the

flight  length  increases .  The  cockpit  airflow  pattern  and

distribution  system  have  an  influence  on  the  low  temperatures  at

pilot  foot  level .  This  is  compounded  by  cooler  air  entering  the

cockpit  from  the  avionics  bay  and  by  temperature  losses  across  the

cockpit  distribution  ducting  from  one  air  outlet  to  another .

Cockpit  foot  warmers  improve  the  situation  as  per  their  basic

function .

To  improve  the  situation  a  package  of  modifications  have  been

developed  which  are  proposed  by  this  Service  Bulletin .

(2)    Objective/Action

This  evaluation  Service  Bulletin  installs  :

—  On  aircraft  MSN  2342  only  :  Thermal  insulation  around  the  forward

pedestal  to  eliminate  cold  surfaces  which  could  cool  the  air  in

the  pedestal  area  by  convection .

—  On  all  aircraft  :

A  new  duct  routing  which  will  drive  the  air  to  a  better  location

to  ensure  a  good  comfort  of  the  pilot's  feet .

—  A  heater  in  the  cockpit  distribution  ducting  just  before  the  foot

level  outlet  (last  outlet  in  the  circuit)  to  allow  the  increase

of  the  outlet  temperature  at  this  level .

(3)    Advantages

Accomplishment  of  this  evaluation  Service  Bulletin  will  provide

information  of  all  evolution  (the  cockpit  air  heating,  thermal

insulation,  foot warmer)  efficiency  in  operational  conditions  for  a

six  months  period .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   3

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(4)    Operational/Maintenance  Consequences

None

## D .     DESCRIPTION

To  accomplish  this  Service  Bulletin  it  is  necessary  to  :

(1)    Config .  01

(a)    Remove  the  equipment  for  access .

(b)    Replace  the  lateral  panels  at  LH  side .

(c)    Replace  the  lateral  panels  at  RH  side .

(d)    Replace  the  panels  assy  at  LH  side .

(e)    Replace  the  panels  assy  at  RH  side .

(f)    Install  the  removed  equipment  for  access .

(2)    Config .  02

(a)    In  the  cockpit,  remove  the  equipment  for  access .

(b)    Install  the  heater  3HE  at  LH  side .

(c)    Install  the  heater  4HE  at  RH  side .

(d)    Modify  the  equipment  and  the  wiring  in  the  rear  panel  120VU .

(e)    Modify  the  equipment  and  the  wiring  between  the  cockpit  and

the  avionics  compartment .

(f)    In  the  RH  side,  modify  the  routing  between  FR2  and  FR3 .

(g)    In  the  LH  side,  modify  the  routing  between  FR2  and  FR3 .

(h)    In  the  avionics  compartment,  modify  the  equipment .

(i)    Modify  the wiring  between  the  equipment  (FINs  3HE  and  4HE)  and

the  avionics  compartment .

(j)    Modify  the  wiring  in  the  relay  box  103VU .

(k)    In  the  cockpit,  install  the  equipment  removed  for  access .

(3)    Config .  03

(a)    In  the  cockpit,  remove  the  equipment  for  access .

(b)    Install  the  heater  3HE  at  LH  side .

(c)    Install  the  heater  4HE  at  RH  side .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   4

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(d)    Modify  the  equipment  and  the  wiring  in  the  rear  panel  120VU .

(e)    Modify  the  equipment  and  the  wiring  between  the  cockpit  and

the  avionics  compartment .

(f)    In  the  RH  side,  modify  the  routing  between  FR2  and  FR3 .

(g)    In  the  LH  side,  modify  the  routing  between  FR2  and  FR3 .

(h)    In  the  avionics  compartment,  modify  the  equipment .

(i)    Modify  the wiring  between  the  equipment  (FINs  3HE  and  4HE)  and

the  avionics  compartment .

(j)    Modify  the  wiring  in  the  relay  box  103VU .

(k)    In  the  cockpit,  install  the  equipment  removed  for  access .

## E .     COMPLIANCE

(1)    Classification

Desirable

(2)    Accomplishment  Timescale

In  accordance  with  operators'  maintenance  schedule .

## F .    APPROVAL

Approved  under  EASA  Design  Organisation  Approval  No .  EASA .21J .031 .

If  an  aircraft  listed  in  the  effectivity  has  a  modification  or  repair

embodied  that  is  not  of  AIRBUS  origin,  and  which  affects  the  content  of

this  Service  Bulletin,  the  operator  is  responsible  for  obtaining

approval  by  its  airworthiness  authority  for  any  adaptation  necessary

before  incorporation  of  the  Service  Bulletin .

## G .     MANPOWER

The  manpower  estimates  given  in  this  Service  Bulletin  are  based  on  the

direct  labor  cost  to  do  the  work .  These  estimates  assume  that  the  work

will  be  done  by  experienced  personnel,  and  may  need  to  be  revised

upwards  to  suit  operator's  circumstances .  The  estimates  do  not  include

the  time  to  prepare,  plan  or  inspect  the  work .  Manufacture  and

procurement  of  parts  and  tools,  drying  times  for  paints,  sealants,  etc,

and  general  administration  work  are  also  not  included .

For  Config .  01

Get  access                                                                                         3 .0

Replacement  of  the  panels                                                              1.5

at  LH  side

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    5

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1.5

1.5

1.5

1 .0

10 .0

6 .0

Replacement  of  the  panels

at  RH  side

Replacement  of  panels  assy

at  LH  side

Replacement  of  panels  assy

at  RH  side

Test

TOTAL  MANHOURS

ELAPSED  TIME  (HOURS)

For  Config .  02  thru  Config .  03

4 .0

16 .0

20.5

20.5

3.0

6.0

2.0

2.0

1.0

6.0

1.0

16.0

7 .0

4 .0

109 .0

58 .0

Get  access

Removal  of  the  equipment

Installation  of  the  heater

at  LH  side

Installation  of  the  heater

at  RH  side

Modif .  equipment  and

wiring  in  120VU

Modif .  equipment  and

wiring  CKPT/AVNCS

Modification  of  the

routing  in  RH  side

Modification  of  the

routing  in  LH  side

Modification  of  equipment

in  AVNCS

Modif .  wiring  between

equipment/AVNCS

Modification  of  the  wiring

in  103VU

Installation  of  the

removed  equipment

Test

Close-up

TOTAL  MANHOURS

ELAPSED  TIME  (HOURS)

For  Config .  01  ,  restoration  of  the  aircraft

10.0

10 .0

6 .0

Restoration  to  initial

conditions

TOTAL  MANHOURS

ELAPSED  TIME  (HOURS)

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   6

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

For  Config .  02  thru  Config .  03  ,  restoration  of  the  aircraft

Restoration  to  initial                                                                109.0

conditions

TOTAL  MANHOURS                                                                             109 .0

ELAPSED  TIME  (HOURS)                                                                   58 .0

## H .    WEIGHT  AND  BALANCE

Config .  01

Manufacturers  Empty  Weight  :  -0 .86  kg  (-1 .90  lb)

Effect  on  Balance                    :  -3 .87  kgm  (-27 .99  lb .ft)

Config .  02  thru  Config .  03

Manufacturers  Empty  Weight  :  +4 .00  kg  (+8 .82  lb)

Effect  on  Balance                    :  +18 .00  kgm  (+130 .19  lb .ft)

## I .     ELECTRICAL  LOAD  DATA

(1)    Direct  Current  (DC)  Load  Changes

Not  changed

(2)    Alternating  Current  (AC)  Load  Changes

(a)    Config .  02  ,  Kit  211908A02R08    and  Config .  03  ,  Kit

211908A05R01

Circuit  Breaker  :  1HE  (New)

Designation  :  CKPT  FOOT  HEATERS

Busbar  :  204XP-C

Nominal  Power  :  288  VA

Electrical  Load  Data  :  +  288  VA

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   7

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

## J .     REFERENCES

Aircraft  Maintenance  Manual  (AMM)

:  06-41- 53  11-00-00  12-34-24

20-28-00  21-26-00  21- 51-00

21- 55-00  21-63-00  22-82-12

22-96-00  22-97-00  23-13-00

23-13-13  23- 51-00  23- 51-12

23-71-00  24-38- 51  24-41-00

25-11- 51  25-13-14  25-13-44

25-15- 51  26-12-00  26-12-12

27-21-42  31-10-00  31- 50-00

31-60-00  32-45-00  33-12-00

33-13-00  33-14-00  34-41-00

34-41-12  34- 52-00  34- 52-12

36-11-00  49-00-00  52-41-00

52- 51-00

Consumable  Material  List  (CML)

Elec .  Std .  Practices  Manual  (ESPM)

Structural  Repair  Manual  (SRM)

:  20-25- 51  20-30-00  20-33-44

20- 52-10  20- 55-00

:  51-42-00  51-43-00  51-44-00

51-46-00  51-49-00

Standards  Manual  (SM)

Service  Bulletin  No .  A320-21-1909

## K .     PUBLICATIONS  AFFECTED

Aircraft  Wiring  List  (AWL)

## L .     INTERCHANGEABILITY/MIXABILITY

Config .  02

DESCRIPTION                    OLD  PART  No .          NEW  PART  No .            INT    MIXABILITY

Not  applicable

Not  applicable

D11311478A00

723-4071-03

D11311898B00

D33110306C00

03

03

Placard

Placard

NOTE  :  For  definitions  of  interchangeability  codes  in  column  INT .  refer

to  ATA  Common  Support  Data  Dictionary  (CSDD),  Chapter  2.

Config .  03

OLD

PART  No .

NEW

PART

No .

DESCRIPTION

INT    MIXABILITY

Placard

Placard

D11311478A00

723-4207-01

D11311898B00

D33110305B00

03

03

Not  applicable

Not  applicable

NOTE  :  For  definitions  of  interchangeability  codes  in  column  INT .  refer

to  ATA  Common  Support  Data  Dictionary  (CSDD),  Chapter  2.

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   8

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

## 2.     MATERIAL  INFORMATION

A .     MATERIAL  —  PRICE  AND  AVAILABILITY

(1)    Material

Customers  with  aircraft  shown  in  the  effectivity  of  this  Service

Bulletin  should  send  a  purchase  order  to  AIRBUS .  Quote  the  number

of  this  Service  Bulletin .  The  address  is  :

AIRBUS

SPARES  SUPPORT  AND  SERVICES

Weg  bein  Jaeger  150

D-22335  HAMBURG

GERMANY

For  ordering  by  internet:  http://spares .airbus .com

For  ordering  by  fax:  +49  40  50  76  25  90

(2)    Price  and  Availability

Kit  211908A02R08

Cost                  :  15,230 .00US  Dollars

Availability  :  120  calendar  days  from  receipt  of  order

The  Kit  availability  given  above  is  the  standard  lead  time  from  the date  of  your  purchase  order .  If  you  require  the  Kit(s)  before  this time,  please  include  a  retrofit  planning  schedule  with  your  order  so  that  we  can  try  to  comply  with  your  requirements .

Kit  211908A03R03

Cost                  :  6,010 .00  US  Dollars

Availability  :  90    calendar  days  from  receipt  of  order

The  Kit  availability  given  above  is  the  standard  lead  time  from  the date  of  your  purchase  order .  If  you  require  the  Kit(s)  before  this time,  please  include  a  retrofit  planning  schedule  with  your  order  so  that  we  can  try  to  comply  with  your  requirements .

Kit  211908A04R00

Cost                  :  37,140 .00US  Dollars

Availability  :  90    calendar  days  from  receipt  of  order                         The  Kit  availability  given  above  is  the  standard  lead  time  from  the date  of  your  purchase  order .  If  you  require  the  Kit(s)  before  this time,  please  include  a  retrofit  planning  schedule  with  your  order  so  that  we  can  try  to  comply  with  your  requirements .                             Kit  211908A05R01

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   9

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Cost                  :  15,170 .00US  Dollars

Availability  :  120  calendar  days  from  receipt  of  order

The  Kit  availability  given  above  is  the  standard  lead  time  from  the

date  of  your  purchase  order .  If  you  require  the  Kit(s)  before  this

time,  please  include  a  retrofit  planning  schedule  with  your  order

so  that  we  can  try  to  comply  with  your  requirements .

## B .     INDUSTRY  SUPPORT  INFORMATION

AIRBUS  will  provide  the  material  at  no  charge  for  the  aircraft  selected for  evaluation  only  and  will  credit  the  manhours  indicated  in  this          Service  Bulletin  at  the  operator's  agreed  in-house  warranty  labor  rate  upon  receipt  of  a  warranty  claim .

## C .     LIST  OF  COMPONENTS

Kit  211908A02R08

ITEM     NEW  PART  No .            QTY  UM   KEYWORD     ITEM     OLD  PART  No .          INT  INST  DISP

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

18

19

20

21

22

23

34

35

36

37

38

D2122872600000

D2122872700000

D2122872700100

D2122872800000

D2122872800100

D2122872900000

D2122873020000

D2122873100000

D2122873100100

D0003005501000

D2122875620000

F2121008920000

ASNA3323-203

F0003078400000

F0003078400100

D5361107200000

D5361107200100

D5392514820000

D5392514820100

D5392514920000

D5391828200000

ABS1114B028D350

ABS1114B028D640

NAS1801-3-7

NAS1801-3-8

MS21042-3

2

1

1

1

1

4

5

1

1

2

9

2

2

1

1

1

1

1

1

2

4

2

2

16

16

16

Tee

Mouthpic

Mouthpic

Outlet

Outlet

Cap

Holder

Support

Support

Cable

Shim

Bolt

Sleeve

Valve

Valve

Panel

Panel

Bracket

Bracket

Bracket

Bracket

Hose

Hose

Screw

Screw

Nut

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   10

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

| ITEM | NEW PART No . | QTY UM | KEYWORD | ITEM | OLD | PART | No . | INT | INST | DISP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

39        ASNA2397-10L

40        NAS1801-3-6

41        NSA5516CA54NJ

42        ABS0901-045

43        NSA5516A06NJ

44        NSA5516A07NJ

45        ABS0901-050

46        NSA5527-03-05

47        ASNA2397-416

48        MS21042-4

49        NAS1801-3-12

50        ABS0901-035

51        ABS1114B020D350

52        NAS1801-3-9

54        D9251587500000

55        D9251587600000

56        ASNA2050DCJ3215

61        ABS5006-2G

81        MS21071L3

82        ASNA2051DCJ2412

100      NSA931320-050

107      D11311898B00

108      ASNA2080G04

115      E0432A06

116      E0343-01

118      NSA5050-3

119      NSA935504-01

120      NSA5527-03-08

121      NAS1801-3-24

122      NSA937901MA2011

123      EN3646A61006BN

124      E0080-02-10C

125      D33110306C00

127      D9550003000197

128      EN3646A61626BZ

129      E0080-01-16C

130      NAS1149F0332P

131      D2122878020000

132      D2122875620200

133      D2122875620400

134      NAS1801-3-13

135      NAS1801-3-15    D9000095220397

46

2

2

8

8

2

10

2

4

2

2

8

2

12

2

2

28

4  M

2

4

1

1

4

2  M

4

8

4

2

2

1

2

2

1

1

1

1

12

1

3

1

2

6

1

Washer

Screw

Clamp

Clamp

Clamp

Clamp

Clamp

Spacer

Washer

Nut

Screw

Clamp

Hose

Screw

Bracket

Bracket

Rivet

Tape

Nut

Rivet

CB

Placard    (107)  D11311478A00

Button

Conduit

Mount

Nut

Mount

Spacer

Screw

Module

CNCTR

BACKSHLL

Placard    (125)  723-4071-03

Panel

CNCTR

BACKSHLL

Washer

Holder

Shim

Shim

Screw

Screw

Bundle

03  *

03  *

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   11

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

ITEM    NEW  PART  No .          QTY  UM  KEYWORD    ITEM    OLD  PART  No .        INT  INST  DISP

Set

Tie

Tie

Tie

Tie

Module

Module

1

900

900

904

900

1

1

D9100095212195

NSA935401-03

NSA935401-04

NSA935401-05

NSA935401-08

NSA937901E1

NSA937901E7

NOTE  :  For  definitions  of  interchangeability  codes  in  column  INT .  refer

to  ATA  Common  Support  Data  Dictionary  (CSDD),  Chapter  2.

NOTE  (*)    Discard

NOTE  :  Refer  to  the  AIRBUS  Standards  Manual  (SM)  if  you  find  part

numbers  of  hardware  components  in  the  related  kit(s)  which  you

cannot  identify  in  the  LIST  OF  COMPONENTS  of  this  Service

Bulletin .  The  SM  will  give  you  the  correct  part  number  to  part

number  relationship .

Kit  211908A03R03

OLD

KEYWORD

INST

DISP

ITEM

PART

No .

INT

ITEM

QTY  UM

NEW  PART  No .

16

17

70

Panel

Panel

Washer

1

1

26

D2701014600000

D2701014600100

ASNA2397JD10L

NOTE  :  Refer  to  the  AIRBUS  Standards  Manual  (SM)  if  you  find  part

numbers  of  hardware  components  in  the  related  kit(s)  which  you

cannot  identify  in  the  LIST  OF  COMPONENTS  of  this  Service

Bulletin .  The  SM  will  give  you  the  correct  part  number  to  part

number  relationship .

Kit  211908A04R00

KEYWORD

OLD

INST

DISP

ITEM

PART

No .

INT

QTY  UM

ITEM

NEW  PART  No .

53

RC0200A00

2        Heater

Kit  211908A05R01

ITEM     NEW  PART  No .            QTY  UM   KEYWORD     ITEM     OLD  PART  No .          INT  INST  DISP

1

2

3

4

5

6

D2122872600000

D2122872700000

D2122872700100

D2122872800000

D2122872800100

D2122872900000

2

1

1

1

1

4

Tee

Mouthpic

Mouthpic

Outlet

Outlet

Cap

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   12

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

| ITEM | NEW PART No . | QTY UM | KEYWORD | ITEM | OLD | PART | No . | INT | INST | DISP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

7

8

9

10

11

12

13

14

15

18

19

20

21

22

23

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

54

55

56

61

81

82

100

107

108

D2122873020000

D2122873100000

D2122873100100

D0003005501000

D2122875620000

F2121008920000

ASNA3323-203

F0003078400000

F0003078400100

D5361107200000

D5361107200100

D5392514820000

D5392514820100

D5392514920000

D5391828200000

ABS1114B028D350

ABS1114B028D640

NAS1801-3-7

NAS1801-3-8

MS21042-3

ASNA2397-10L

NAS1801-3-6

NSA5516CA54NJ

ABS0901-045

NSA5516A06NJ

NSA5516A07NJ

ABS0901-050

NSA5527-03-05

ASNA2397-416

MS21042-4

NAS1801-3-12

ABS0901-035

ABS1114B020D350

NAS1801-3-9

D9251587500000

D9251587600000

ASNA2050DCJ3215

ABS5006-2G

MS21071L3

ASNA2051DCJ2412

NSA931320-050

D11311898B00

ASNA2080G04

5

1

1

2

9

2

2

1

1

1

1

1

1

2

4

2

2

16

16

16

46

2

2

8

8

2

10

2

4

2

2

8

2

12

2

2

28

4  M

2

4

1

1

4

Holder

Support

Support

Cable

Shim

Bolt

Sleeve

Valve

Valve

Panel

Panel

Bracket

Bracket

Bracket

Bracket

Hose

Hose

Screw

Screw

Nut

Washer

Screw

Clamp

Clamp

Clamp

Clamp

Clamp

Spacer

Washer

Nut

Screw

Clamp

Hose

Screw

Bracket

Bracket

Rivet

Tape

Nut

Rivet

CB

(107)  D11311478A00

Placard

Button

03  *

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   13

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

Conduit

Mount

Nut

Mount

Spacer

Screw

Module

CNCTR

BACKSHLL

Placard    (125)  723-4207-01

Panel

CNCTR

BACKSHLL

Washer

Holder

Shim

Shim

Screw

Screw

Bundle

Set

Tie

Tie

Tie

Tie

Module

Module

2  M

4

8

4

2

2

1

2

2

1

1

1

1

12

1

3

1

2

6

1

1

900

900

904

900

1

1

03  *

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

KEYWORD

DISP

OLD

INST

ITEM

QTY  UM

PART

No .

INT

ITEM

NEW  PART  No .

115      E0432A06

116      E0343-01

118      NSA5050-3

119      NSA935504-01

120      NSA5527-03-08

121      NAS1801-3-24

122      NSA937901MA2011

123      EN3646A61006BN

124      E0080-02-10C

125      D33110305B00

127      D9550003000295

128      EN3646A61626BZ

129      E0080-01-16C

130      NAS1149F0332P

131      D2122878020000

132      D2122875620200

133      D2122875620400

134      NAS1801-3-13

135      NAS1801-3-15    D9000095220397 D9100095212195 NSA935401-03    NSA935401-04    NSA935401-05    NSA935401-08    NSA937901E1      NSA937901E7

NOTE  :  For  definitions  of  interchangeability  codes  in  column  INT .  refer

to  ATA  Common  Support  Data  Dictionary  (CSDD),  Chapter  2.

NOTE  (*)    Discard

NOTE  :  Refer  to  the  AIRBUS  Standards  Manual  (SM)  if  you  find  part

numbers  of  hardware  components  in  the  related  kit(s)  which  you

cannot  identify  in  the  LIST  OF  COMPONENTS  of  this  Service

Bulletin .  The  SM  will  give  you  the  correct  part  number  to  part

number  relationship .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   14

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

## D .     LIST  OF  MATERIALS  —  OPERATOR  SUPPLIED

(1)    Consumable  Materials

| DESCRIPTION | REFERENCE TO CML MAT . No . | QTY | PER | A/C | INST | DISP |
| --- | --- | --- | --- | --- | --- | --- |

Pure  Mineral  Vaseline  Or

Petro-Latum

Electrical  Bonding  Coating

(Blue  Color)

Dry  Cleaning  Solvent

| (Varsol/White Tape Hook and ABS1133A025 Self Adhesive | Spirit) Loop ABS1133D025 |
| --- | --- |

rivet  ASNA2050DCJ3215

rivet  ASNA2050DCJ4828

Rivet  ASNA2051DCJ2412

Button  Rivet  Plastic

ASNA2080G01

04-012

07-001

11-002

None

None

None

None

None

None

As

As

As

As

As

required

required

required

required

required

68

12

8

4

(2)    Components

## None

E .     PARTS  TO  BE  RE— IDENTIFIED  BY  THE  OPERATOR

## None

F .    TOOLING  —  PRICE  AND  AVAILABILITY

None

## G .     SPECIAL  TOOLS

None

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   15

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

This  Page  Intentionally  Left  Blank

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   16

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

# 3.    ACCOMPLISHMENT  INSTRUCTIONS

## A .     GENERAL

WARNING  :  MAKE  SURE  THAT  YOU  OBEY  ALL  THE  WARNINGS  AND  ALL  THE  CAUTIONS

INCLUDED  IN  THE  REFERENCED  PROCEDURES .

CAUTION  :  ALWAYS  OBEY  THE  PRECAUTIONS  THAT  FOLLOW  TO  KEEP  ELECTRICAL

WIRING  IN  A  SATISFACTORY  CONDITION  (ELECTRICALLY  AND

MECHANICALLY  SERVICEABLE) .  WHEN  YOU  DO  MAINTENANCE  WORK,

REPAIRS  OR  MODIFICATIONS,  ALWAYS  KEEP  ELECTRICAL  WIRING,

COMPONENTS  AND  THE  WORK  AREA  AS  CLEAN  AS  POSSIBLE,  TO  DO  THIS

:

—  PUT  PROTECTION,  SUCH  AS  PLASTIC  SHEETING,  CLOTHS,  ETC;  AS

NECESSARY  ON  WIRING  AND  COMPONENTS .

—  REGULARLY  REMOVE  ALL  SHAVINGS,  UNWANTED  MATERIAL  AND  OTHER

CONTAMINATION .

THESE  PRECAUTIONS  WILL  DECREASE  THE  RISK  OF  CONTAMINATION  AND

DAMAGE  TO  THE  ELECTRICAL  WIRING  INSTALLATION .

IF  THERE  IS  CONTAMINATION  REFER  TO  ESPM  20- 55-00 .

(1)    Preparation

(a)    Config .  01

1    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Multipurpose  Control  &  Display Unit  (MCDU)  (3CA1,3CA2  and  3CA3)  (Refer  to  AMM  22-82-12      Page  block  401) .

2    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Radio  Management  Panel  (RMP) (1RG1,  1RG2  and  1RG3)  (Refer  to  AMM  23-13-13  Page  block  401) .

3    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Audio  Control  Panel  (ACP)      (2RN1,  2RN2,  2RN3,  2RN4  and  2RN5)  (Refer  to  AMM  23- 51-12 Page  block  401) .

4    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Weather  Radar  control  unit

(WXR)  (3SQ)  (Refer  to  AMM  34-41-12  Page  block  401) .

5    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Air  Traffic  Control/  Traffic Alert  and  Collision  Avoidance  System  (ATC/TCAS)  control  unit  (3SH)  (Refer  to  AMM  34- 52-12  Page  block  401) .

6    Open,  safety  and  tag  these  circuit  breakers  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   17

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

LOCATION

PANEL

FIN

DESIGNATION

49VU

122VU

122VU

121VU

1LE

2LE

4LF

5TV

08  H

04  Z

04  Y

15  K

LIGHTING/EMER  LT/CKPT/DOME

LIGHTING/FLOOD/CTR  INST/PNL

LIGHTING/INSTL  LT/MAIN  INST/PNL

AND/PED

AIDS  &  RCDR/FDIMU

(b)    Config .  02  thru  Config .  03

1    Make  sure  that  the  aircraft  is  electrically  grounded  (Refer

to  AMM  12- 34-24  Page  block  201) .

2    Put  the  access  platform(s)  in  position .

3    Open  the  access  doors  811  and  822  (Refer  to  AMM  52-41-00

Page  block  001) .

4    In  the  cockpit,  remove  the  sidewall  and  ceiling  panels

211BW,  212BW,  211HC,  212FC,  212KC,  212JW  and  212HW  (Refer

to  AMM  06-41- 53  Page  block  001) .

5    Remove  the  textile  floor  covering  (Refer  to  AMM  25-13-44

Page  block  401) .

6    Remove  the  panel  Item  (141)  on  the  left  of  the  panel  120VU

(Refer  to  Figure  11  ,  Sheet  1)  for  access .

7    Make  sure  that  the  external  connector(s)  is  (are)  not

connected  to  the  aircraft  receptacle(s)  EXT  PW .

8    Make  sure  that  there  is  (are) warning  notice(s)  on  external

power  receptacle  to  tell  persons  not  to  connect  an  external

power  source .

9    Make  sure  that  the  EMER  EXIT  LT  pushbutton  switch  on  25VU

is  on  OFF  position .

10  In  the  cocpkit,  on  the  overhead  panel  35VU  release  BAT

pushbutton  switches .

11  Put  a  warning  notice  on  refuel/defuel  panel  800VU  to  tell

persons  no  to  operate  the  NORMAL  BATTERY  switch .

12  Put  a  warning  notice  in  the  cockpit  to  tell  persons  not  to

start  the  Auxiliary  Power  Unit  (APU) .

13  Put  a  warning  notice  in  the  cockpit  to  tell  persons  not  to

start  the  ENG  1,  2.

14  Battery  disconnection  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   18

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  For  job  set-up,  refer  to  the  removal/installation  of  the

batteries  (Refer  to  AMM  24- 38- 51  Page  block  401) .

—  Turn  the  knurled  nut  a  quarter  turn .

—  Disconnect  the  connectors .

—  Put  blanking  caps  on  the  disconnected  electrical

connectors .

(2)    Standard  Practices

(a)    For  the  specification  of  the  material  numbers  (Mat .  No .),

refer  to  CML .

(b)    Torque  the  standard  threaded  fasteners  (Refer  to  SRM

51-49-00) .

(c)    For  alternative  and  substitute  fasteners,  refer  to  SRM

51-43-00 .

(d)    Drill  and  deburr  the  holes  (Refer  to  SRM  51-44-00) .

(e)    To  countersink  the  holes,  refer  to  SRM  51-46-00 .

(f)    Remove/install  the  fasteners  (Refer  to  SRM  51-42-00) .

(g)    Do  the  electrical  bonding  (Refer  to  AMM  20-28-00  Page  block

201)  with  Pure  Mineral  Vaseline  Or  Petro-Latum  (Mat  No .

04-012)    and  Electrical  Bonding  Coating  (Blue  Color)  (Mat  No .

07-001)  .

(h)    Obey  the  instructions  for  the  general  wiring  installation

(Refer  to  ESPM  20- 30-00) .

(i)    To  install  new  wires,  cut  them  to  the  necessary  length,  crimp

the  terminals  and  connect  them .

(j)    Route  the  wires  with  the  wires  that  are  in  the  aircraft .

(k)    Use  existing  spare  conduits  to  route  wires  in  locations

difficult  to  get  access  to .

(l)    Do  a  continuity  test  of  the  modified  and  new  wires .

(m)    For  the  cable  tie  application  and  tooling,  refer  to  ESPM

20- 33-44  and  ESPM  20-25- 51 .

(n)    Clean  the  surfaces  at  location  of  the  placards  with  Dry

Cleaning  Solvent  (Varsol/White  Spirit)  (Mat  No .  11-002)  .

(o)    Bond  the  placards  (Refer  to  AMM  11-00-00  Page  block  201) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   19

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

## B .     MODIFICATION

(1)    Config .  01

(a)    Remove  the  equipment  for  access .

Refer  to  AMM  22-82-12  Page  block  401

Refer  to  AMM  23-13-13  Page  block  401

Refer  to  AMM  23- 51-12  Page  block  401

Refer  to  AMM  34-41-12  Page  block  401

1    Remove  the  MCDU  (3CA1  and  3CA2)  (Refer  to AMM  22-82-12  Page

block  401) .

2    Remove  the  RMP  (1RG1  and  1RG2)  (Refer  to  AMM  23-13-13  Page

block  401) .

3    Remove  the  ACP  (2RN1  and  2RN2)  (Refer  to  AMM  23- 51-12  Page

block  401) .

4    Remove  the  weather  radar  CTL  unit  (Refer  to  AMM  34-41-12

Page  block  401) .

5    Remove  the  panel  111VU .

6    Remove  the  panel  112VU .

(b)    Replace  the  lateral  panels  at  LH  side .

Refer  to  Figure  3  ,  Sheets  1    and  2

Refer  to  Kit  211908A03R03

1    Remove  :

1        Panel  foam

1         Strip  board

machined

1         Strip  board

machined

attached  with  :

3        Screw

6        Screw

2        Screw

9        Washer

and

1        Strip  board

machined

Item  (86)    Retain

Item  (63)    Retain

Item  (67)    Retain

Item  (88)    Retain

Item  (71)    Retain

Item  (89)    Retain

Item  (70)    Discard

Item  (65)    Retain

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   20

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

attached  with  :

4

4

Screw

Washer

Item  (69)    Retain

Item  (70)    Discard

2    Install  :

1         Lateral  panel           D2701014600000    Item  16

assy

1        Panel  foam                                                 Item  (86)

with  :

3        Screw                                                           Item  (88)

6        Screw                                                           Item  (71)

4        Screw                                                           Item  (69)

2        Screw                                                           Item  (89)

13        Washer                        ASNA2397JD10L      Item  70

NOTE  :  Items  (69),  (71),  (86),  (88)  and  (89)  were  retained

at  removal .

(c)    Replace  the  lateral  panels  at  RH  side .

Refer  to  Figure  3  ,  Sheets  1    and  2

1    Remove  :

1        Panel  foam

1         Strip  board

machined

1         Strip  board

machined

attached  with  :

3        Screw

6        Screw

2        Screw

9        Washer

and

1         Strip  board

machined

attached  with  :

4        Screw

4        Washer

2    Install  :

Item  (87)    Retain

Item  (64)    Retain

Item  (68)    Retain

Item  (88)    Reatin

Item  (71)    Retain

Item  (89)    Retain

Item  (70)    Discard

Item  (66)    Retain

Item  (69)    Retain

Item  (70)    Discard

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   21

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1        Panel  foam                                                 Item  (87)

1         Lateral  panel           D2701014600100    Item  17

assy

with  :

3        Screw                                                           Item  (88)

6        Screw                                                           Item  (71)

4        Screw                                                           Item  (69)

2        Screw                                                           Item  (89)

13        Washer                        ASNA2397JD10L      Item  70

NOTE  :  Items  (69),  (71),  (87),  (88)  and  (89)  were  retained

at  removal .

(d)    Install  the  removed  equipment  for  access .

Refer  to  AMM  22-82-12  Page  block  401

Refer  to  AMM  23-13-13  Page  block  401

Refer  to  AMM  23- 51-12  Page  block  401

Refer  to  AMM  34-41-12  Page  block  401

1    Install  the  MCDU  (3CA1  and  3CA2)  (Refer  to  AMM  22-82-12

Page  block  401) .

2    Install  the  RMP  (1RG1  and  1RG2)  (Refer  to AMM  23-13-13  Page

block  401) .

3    Install  the ACP  (2RN1  and  2RN2)  (Refer  to AMM  23- 51-12  Page

block  401) .

4    Install  the  weather  radar  CTL  unit  (Refer  to  AMM  34-41-12

Page  block  401) .

5    Install  the  panel  111VU .

6    Install  the  panel  112VU .

(2)    Config .  02

(a)    In  the  cockpit,  remove  the  equipment  for  access .

Refer  to  AMM  25-11- 51

Refer  to  AMM  27-21-42

refer  to  AMM  25-13-14

Refer  to  AMM  25-15- 51

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   22

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1    Remove  the  captain  and  first  officer  seats  (3MS  and  4MS)

(Refer  to  AMM  25-11- 51  Page  block  401) .

2    Remove  the  captain  and  first  officer  pedals  (Refer  to  AMM

27-21-42  Page  block  401) .

3    Remove  the  foot  rest  assembly  (Refer  to  AMM  25-13-14  Page

block  401) .

4    Remove  the  captain  and  first  officer  foot  warmers  (2DR1,

2DR2,  2DR3  and  2DR4)  (Refer  to  AMM  25-15- 51  Page  block

401) .

5    Remove  the  duct  Item  (90)  for  access .

Refer  to  Figure  1  ,  Sheet  1

a    Remove  :

1        Duct                                                             Item  (90)    Retain

attached  with  :

1        Clamp                                                           Item  (91)    Retain

1        Clamp                                                           Item  (92)    Retain

2        Clamp                                                           Item  (95)    Retain

1        Sleeve                                                         Item  (96)    Retain

2        Clamp                                                           Item  (93)    Retain

1        Sleeve                                                         Item  (94)    Retain

(b)    Install  the  tape  adhesive  Item  61  on  the  panel  assy  at  LH

side .

Refer  to  Figure  4  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

(c)    Install  the  tape  adhesive  Item  61  on  the  panel  assy  at  RH

side .

Refer  to  Figure  4  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   23

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(d)    Install  the  heater  3HE  at  LH  side .

Refer  to  Figure  5  ,  Sheet  1

Refer  to  Figure  6  ,  Sheets  1    thru  5

Refer  to  Figure  7  ,  Sheets  1    thru  4

Refer  to  Figure  8  ,  Sheets  1    and  2

Refer  to  Kit  211908A02R08

1    Replace  the  cabin  floor  panel .

Refer  to  Figure  5  ,  Sheet  1

a    Remove  the  cabin  floor  Item  (18) .

—  Disconnect  the  connector  1025VC  for  access .

—  Remove  :

1        Cabin  floor                                               Item  (18)    Retain

attached  with  :

6        Screw                                                           Item  (29)    Retain

b    Install  the  cabin  floor  Item  18.

—  Install  :

1        Panel  -  floor          D5361107200000    Item  18

with  :

6        Screw                                                           Item  (29)

NOTE  :  The  screw  Item  (29)  was  retained  at  removal .

—  Connect  the  connector  1025VC  already  disconnected  for

access .

2    Install  the  ducts  and  the  heater  3HE .

| Refer Refer Refer Refer Refer | to to to to to | Figure Figure Figure Figure Figure | 1 , Sheet 1 2 , Sheets 1 and 2 6 , Sheets 1 thru 5 7 , Sheets 1 thru 4 8 , Sheets 1 and 2 |
| --- | --- | --- | --- |

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   24

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

a    Remove  the  duct  Item  (74) .

Refer  to  Figure  6  ,  Sheet  1

—  Remove  :

1        Duct                                                             Item  (74)    Retain

attached  with  :

2        Clamp                                                           Item  (73)    Retain

and

1         Base  assy                                                      Item  (83)    Retain

mounting

1        Outlet  air                                                 Item  (84)    Retain

attached  with  :

4        Screw                                                           Item  (85)    Retain

b    Disconnect  the  cable  Item  (80) .

Refer  to  Figure  6  ,  Sheet  1

—  Disconnect  :

1        Cable                                                           Item  (80)    Retain

attached  with  :

1        Bolt                                                             Item  (75)    Retain

1        Clamp                                                           Item  (77)    Retain

1        Screw                                                           Item  (79)    Retain

1        Washer                                                         Item  (78)    Discard

1        Nut                                                               Item  (76)    Discard

—  Attach  temporarily  the  cable  Item  (80)  on  the

structure  with  :

1        Tie-Cable                  NSA935401-05

c    Install  a  tie-cable  on  the  flap  of  the  air  outlet  (only

on  2342  ) .

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1        Tie-Cable                  NSA935401-05

NOTE  :  The  tie-cable  is  installed  to  lock  the  flap  of

the  air  outlet  in  close  position .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   25

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@

Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

d    Install  the  tape-adhesive  Item  61.

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

e    Install  the  brackets  Item  54,

the  structure .

Refer  to  Figure  6  ,  Sheets  2

Refer  to  Figure  7  ,  Sheets  1

Refer  to  Figure  8  ,  Sheets  1

—  Install  the  bracket  Item  54

55,  20,  22,  23,  7  and  8  on

thru  5

thru  4

and  2

on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  54  PN

D9251587500000  on  the  structure  as  shown  on  Figure  7

,  Sheet  2   ,  detail  B .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  7  ,  Sheet  2  .

.  Install  :

1        Bracket                      D9251587500000    Item  54

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  55  on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  55  PN

D9251587600000  on  the  structure  as  shown  on  Figure  7

,  Sheet  2    detail  C .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587600000    Item  55 with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   26

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  20,  Item  22  and  Item  23  on

the  cabin  floor .

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Temporarily  put  in  position  the  brackets  Item  20  PN

D5392514820000,  Item  22  PN  D5392514920000,  Item  23

PN  D5391828200000  on  the  structure  as  shown  on

Figure 7  ,  Sheet  3  ,  detail  D .

.  Drill  and  deburr  eight  holes  on  the  structure  to  the

dimensions  given  in  Figure  7  ,  Sheet  3  ,  detail  D .

.  Install  :

1        Bracket                      D5392514820000    Item  20

with  :

3        Rivet                          ASNA2050DCJ3215  Item  56

and

1        Bracket                      D5392514920000    Item  22

with  :

3        Rivet                          ASNA2050DCJ3215  Item  56

and

1        Bracket                      D5391828200000    Item  23

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  23  between  FR2  and  FR3  on  the

under  floor .

Refer  to  Figure  7  ,  Sheets  1    and  4

.  Temporarily  put  in  position  the  bracket  Item  23  PN

D5391828200000  on  the  structure  as  shown  on  Figure  7

,  Sheet  4   ,  detail  F .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  7  ,  Sheet  4  .

.  Install  :

1        Bracket                      D5391828200000    Item  23

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   27

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2

3

6

6

6

1

2

2

1

2

with  :

Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  brackets  Item  7,  Item  8  and  their  related

shims  on  the  floor  panel .

Refer  to  Figure  6  ,  Sheets  2    thru  5

.  Install  :

Holder                        D2122873020000    Item  7

Shim                            D2122875620000    Item  11

with  :

| Screw Washer and Support assy with : Screw Washer — Install the | NAS1801-3-8 Item 37 ASNA2397-10L Item 39 D2122873100000 Item 8 NAS1801-3-6 Item 40 ASNA2397-10L Item 39 nut Item 81 on the cabin floor . |
| --- | --- |

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Drill  and  deburr  one  hole  on  the  cabin  floor  to  the

dimensions  given  in  Figure  7  ,  Sheet  3  ,  detail  D .

.  Temporarily  put  in  position  the  nut  Item  81  PN

MS21071L3  on  the  cabin  floor  as  shown  on  Figure  7   ,

Sheet 3  ,  detail  D .

.  Drill  and  deburr  two  holes  on  the  cabin  floor  to  the

dimensions  given  in  Figure  7  ,  Sheet  3  ,  detail  D .

.  Install  :

Nut                              MS21071L3              Item  81

with  :

Rivet-Solid              ASNA2051DCJ2412  Item  82

—  As  shown  on  Figure  5,  Sheet  4,  detail  G,  bond  the

placard  HEATER  3HE  supplied  in

Placard  set              D9100095212195

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   28

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

f    Install  the  grounding  points  and  the  placards  203VN,

205VN,  207VN .

Refer  to  Figure  8  ,  Sheets  1    and  2

—  Drill  three  holes  on  the  structure  to  a  diameter  of

5 .2mm  (0 .204in .)  as  shown  on  Figure  7,  Sheet  1.

—  On  the  structure,  do  a  stripping  to  the  dimension

given  in  Figure  7,  Sheet  1.

—  As  shown  on  Figure  7,  Sheet  2,  detail  A,  bond  the

placards  203VN,  205VN,  207VN,  supplied  in  :

Placard  set              D9100095212195

g

Install  the  ducts  and  the  heater  3HE .

Refer  to  Figure  6  ,  Sheets  2    thru  5

—  Install  :

| 1 | Hose with : | ABS1114B020D350 | Item | 51 |
| --- | --- | --- | --- | --- |
| 1 | Clamp and | ABS0901-035 | Item | 50 |
| 1 | Tee - Helded | D2122872600000 | Item | 1 |
| 1 | Sleeve with : | ASNA3323-203 | Item | 13 |
| 3 | Clamp and | ABS0901-035 | Item | 50 |
| 1 | Heater | RC0200A00 | Item | 53 |
| 1 | Valve 3 ways | F0003078400000 | Item | 14 |
| 1 | Valve with : | D2122872700000 | Item | 2 |
| 4 | Screw | NAS1801-3-9 | Item | 52 |
| 4 | Screw | NAS1801-3-7 | Item | 36 |
| 8 | Washer | ASNA2397-10L | Item | 39 |
| 8 | Nut and | MS21042-3 | Item | 38 |
| 1 | Hose | ABS1114B028D640 | Item | 35 |

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   29

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

with  :

2        Clamp                          ABS0901-045          Item  42

NOTE  :  Attach  the  hose  Item  35  on  the  existing  duct

with  Tape  Hook  and  Loop  ABS1133A025    and  Self

Adhesive  ABS1133D025  .

and

1        Outlet  -  blowing    D2122872800000    Item  4

with  :

3        Clamp                          ABS0901-050          Item  45

and

2        Cap  by  pass              D2122872900000    Item  6

with  :

2        Clamp                          ABS0901-050          Item  45

and

1        Hose                            ABS1114B028D350  Item  34

with  :

2        Clamp                          ABS0901-045          Item  42

1        Clamp                          NSA5516CA54NJ      Item  41

2        Screw                          NAS1801-3-7          Item  36

2        Washer                        ASNA2397-10L        Item  39

h    Install  the  cable  Item  10.

Refer  to  Figure  6  ,  Sheets  2    and  3

—  Install  :

1        Cable                          D0003005501000    Item  10

with  :

2

1

2

1

1

5

4

1

Screw

Screw

Screw

Bolt

Spacer

Washer

Clamp

Clamp

NAS1801-3-7

NAS1801-3-12

NAS1801-3-8

F2121008920000

NSA5527-03-05

ASNA2397-10L

NSA5516A06NJ

NSA5516A07NJ

Item

Item

Item

Item

Item

Item

Item

Item

36

49

37

12

46

39

43

44

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   30

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2

1

Washer

Nut

ASNA2397-416

MS21042-4

Item  47

Item  48

(e)    Install  the  heater  4HE  at  RH  side .

Refer  to  Figure  5  ,  Sheet  1

Refer  to  Figure  6  ,  Sheets  1    thru  5

Refer  to  Figure  7  ,  Sheets  1    thru  4

Refer  to  Figure  9  ,  Sheets  1    and  2

Refer  to  Kit  211908A02R08

Refer  to  Kit  211908A03R03

1    Replace  the  cabin  floors  panel .

Refer  to  Figure  5  ,  Sheet  1

a    Remove  the  cabin  floor  Item  (19) .

—  Remove  :

1        Cabin  floor                                               Item  (19)    Retain

attached  with  :

6        Screw                                                           Item  (29)    Retain

b    Install  the  cabin  floor  Item  19.

—  Install  :

1        Panel  -  floor          D5361107200100    Item  19

with  :

6        Screw                                                           Item  (29)

NOTE  :  The  screw  Item  (29)  was  retained  at  removal .

2    Install  the  ducts  and  the  heater  4HE .

Refer  to  Figure  2  ,  Sheets  1 Refer  to  Figure  6  ,  Sheets  1 Refer  to  Figure  7  ,  Sheets  1 Refer  to  Figure  9  ,  Sheets  1

a    Remove  the  duct  Item  (74) .

and  2

thru  5

thru  4

and  2

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   31

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  6  ,  Sheet  1

—  Remove  :

1        Duct

attached  with  :

2        Clamp

and

1         Base  assy

mounting

1        Outlet  air

attached  with  :

4        Screw

b    Disconnect  the  cable  Item  (80) .

Refer  to  Figure  6  ,  Sheet  1

—  Disconnect  :

1        Cable

attached  with  :

1        Bolt

1        Clamp

1        Screw

1        Washer

1        Nut

Item  (74)    Retain

Item  (73)    Retain

Item  (83)    Retain

Item  (84)    Retain

Item  (85)    Retain

Item  (80)    Retain

Item  (75)    Retain

Item  (77)    Retain

Item  (79)    Retain

Item  (78)    Discard

Item  (76)    Discard

—  Attach  temporarily  the  cable  Item  (80)  on  the

structure  with  :

1        Tie-Cable                  NSA935401-05

c    Install  a  tie-cable  on  the  flap  of  the  air  outlet .

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1        Tie-Cable                  NSA935401-05

NOTE  :  The  tie-cable  is  installed  to  lock  the  flap  of

the  air  outlet  in  close  position .

d    Install  the  tape-adhesive  Item  61  (only  on  the  2342  ) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   32

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@

Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

e    Install  the  brackets  Item  54,

the  structure .

Refer  to  Figure  6  ,  Sheets  1

Refer  to  Figure  7  ,  Sheets  1

Refer  to  Figure  9  ,  Sheets  1

—  Install  the  bracket  Item  54

55,  21,  22,  23,  7  and  9  on

thru  5

thru  4

and  2

on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  54  PN

D9251587500000  on  the  structure  as  shown  on  Figure

6,  Sheet  2  on  detail  B .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587500000    Item  54

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  55  on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  55  PN

D9251587600000  on  the  structure  as  shown  on  Figure

6,  Sheet  2  on  detail  C .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587600000    Item  55

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   33

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  Install  the  bracket  Item  21,  Item  22,  Item  23  on  the

cabin  floor .

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Temporarily  put  in  position  the  brackets  Item  21  PN

D5392514820100,  Item  22  PN  D5392514920000,  Item  23

PN  D5391828200000  on  the  structure  as  shown  on

Figure  6,  Sheet  3  on  detail  D .

.  Drill  and  deburr  eight  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  3.

1

3

1

3

1

2

1

.  Install  :

Bracket                      D5392514820100    Item  21

with  :

Rivet                          ASNA2050DCJ3215  Item  56

and

Bracket                      D5392514920000    Item  22

with  :

Rivet                          ASNA2050DCJ3215  Item  56

and

Bracket                      D5391828200000    Item  23

with  :

Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  23  between  FR2  and  FR3  on  the

under  floor .

Refer  to  Figure  7  ,  Sheets  1    and  4

.  Temporarily  put  in  position  the  bracket  Item  23  PN

D5391828200000  on  the  structure  as  shown  on  Figure

6,  Sheet  4  on  detail  F .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  4.

.  Install  :

Bracket                      D5391828200000    Item  23

with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   34

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  brackets  Item  7,  Item  9  ,  Item  131  and

their  related  shims  on  the  floor  panel .

Refer  to  Figure  6  ,  Sheets  2    thru  5

.  Install  :

2

1

3

3

Holder

Holder

Shim

Shim

7

131

11

132

Item

Item

Item

Item

D2122873020000

D2122878020000

D2122875620000

D2122875620200

with  :

Screw

Washer

6

6

135

39

Item

Item

NAS1801-3-15

ASNA2397-10L

and

D2122873100100

D2122875620400

1

1

9

133

Item

Item

Support  assy

Shim

with  :

Screw

Washer

2

2

134

39

Item

Item

NAS1801-3-13

ASNA2397-10L

—  Install  the  nut  Item  81  on  the  cabin  floor .

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Drill  and  deburr  one  hole  on  the  cabin  floor  to  the

dimensions  given  in  Figure  6,  Sheet  3,  detail  D .

.  Temporarily  put  in  position  the  nut  Item  81  PN

MS21071L3  on  the  cabin  floor  as  shown  on  Figure  6,

Sheet  3,  detail  D .

.  Drill  and  deburr  two  holes  on  the  cabin  floor  to  the

dimensions  given  in  Figure  6,  Sheet  3,  detail  D .

.  Install  :

1        Nut                              MS21071L3              Item  81

with  :

2        Rivet-Solid              ASNA2051DCJ2412  Item  82

—  As  shown  on  Figure  5,  Sheet  4,  detail  G,  bond  the

placard  HEATER  4HE  supplied  in  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   35

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Placard  set              D9100095212195

f    Install  the  grounding  points  and  the  placards  204VN,

206VN,  208VN .

Refer  to  Figure  9  ,  Sheets  1    and  2

—  Drill  three  holes  on  the  structure  to  a  diameter  of

5 .2mm  (0 .204in .)  as  shown  on  Figure  8,  Sheet  1.

—  On  the  structure,  do  a  stripping  to  the  dimension

given  in  Figure  8,  Sheet  1.

—  As  shown  on  Figure  8,  Sheet  2,  detail  A,  bond  the

placards  204VN,  206VN,  208VN,  supplied  in  :

Placard  set              D9100095212195

g

Install  the  ducts  and  the  heater  4HE .

Refer  to  Figure  6  ,  Sheets  2    thru  5

—  Install  :

1        Hose                            ABS1114B020D350  Item  51

with  :

1        Clamp                          ABS0901-035          Item  50

and

1        Tee  -  Helded            D2122872600000    Item  1

1        Sleeve                        ASNA3323-203        Item  13

with  :

3        Clamp                          ABS0901-035          Item  50

and

1        Heater                        RC0200A00              Item  53

1        Valve  3  ways            F0003078400100    Item  15

1        Valve                          D2122872700100    Item  3

with  :

4        Screw                          NAS1801-3-9          Item  52

4        Screw                          NAS1801-3-7          Item  36

8        Washer                        ASNA2397-10L        Item  39

8        Nut                              MS21042-3              Item  38

and

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   36

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1        Hose                            ABS1114B028D640  Item  35

with  :

2        Clamp                          ABS0901-045          Item  42

NOTE  :  Attach  the  hose  Item  35  on  the  existing  duct

with  Tape  Hook  and  Loop  ABS1133A025    and  Self

Adhesive  ABS1133D025  .

and

1        Outlet  -  blowing    D2122872800100    Item  5

with  :

3        Clamp                          ABS0901-050          Item  45

and

2        Cap  by  pass              D2122872900000    Item  6

with  :

2        Clamp                          ABS0901-050          Item  45

and

1        Hose                            ABS1114B028D350  Item  34

with  :

2        Clamp                          ABS0901-045          Item  42

1        Clamp                          NSA5516CA54NJ      Item  41

2        Screw                          NAS1801-3-7          Item  36

2        Washer                        ASNA2397-10L        Item  39

h    Install  the  cable  Item  10.

Refer  to  Figure  6  ,  Sheets  2    and  Sheet  3

—  Install  :

1        Cable                          D0003005501000    Item  10

with  :

2

1

2

1

1

5

Screw

Screw

Screw

Bolt

Spacer

Washer

NAS1801-3-7

NAS1801-3-12

NAS1801-3-8

F2121008920000

NSA5527-03-05

ASNA2397-10L

Item

Item

Item

Item

Item

Item

36

49

37

12

46

39

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   37

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

4        Clamp

1        Clamp

2        Washer

1        Nut

(f)    Modify  the  equipment

Refer  to  Figure  11  ,

Refer  to  Figure  18

Refer  to  Figure  23

NSA5516A06NJ

NSA5516A07NJ

ASNA2397-416

MS21042-4

and  the  wiring  in

Sheets  1    and  2

Item  43

Item  44

Item  47

Item  48

the  rear  panel  120VU .

Refer  to  Kit  211908A02R08

1    Install  the  circuit  breaker  1HE  and  the  related  placard

44LM .

Refer  to  Figure  11  ,  Sheets  1    and  2

a    From  the  rear  C/B  panel  122VU,  remove  :

FIN  44LM

1        Placard                      D11311478A00        Item  (107)  Discard

attached  with  :

4        Button  Rivet                                             Item  (108)  Discard

Plastic

b    On  the  rear  C/B  panel  122VU,  install  :

FIN  44LM

1

4

Placard

with  :

Button  Rivet

Plastic

D11311898B00

ASNA2080G04

Item  107

Item  108

or  with  :  Button  Rivet  Plastic  ASNA2080G01

c    In  the  rear  C/B  panel  122VU,  install  at  the  location V23

:

FIN  1HE

1        Circuit  Breaker      NSA931320-050      Item  100

eqipped  with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   38

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2

2

Screw

Washer-Locking

E0736N08-01

ASNA2553-0401

Item  109

Item  110

d    Near  the  circuit  breaker,  bond  the  placard  1HE,  supplied

in  :

Placard  set              D9100095212195

2    Modify  the  wiring  in  the  rear  panel  120VU .

Refer  to  Figure  18

Refer  to  Figure  23

a    Install  the wires  as  shown  on  the wiring  diagram,  Figure 18  ,  Sheet  1    and  as  given  in  the  Hook-up  Chart  lines  :

—  1  and  2  (Refer  to  Figure  23  ,  Sheet  1)  supplied  in  :

Bundle                        D9000095220397

b    Attach  the  wires  with  :

150        Tie-Cable                  NSA935401-03

150        Tie-Cable                  NSA935401-04

150        Tie-Cable                  NSA935401-05

150        Tie-Cable                  NSA935401-08

(g)    Modify  the  equipment  and  the  wiring  between  the  cockpit  and

the  avionics  compartment .

Refer  to  Figure  10

Refer  to  Figure  12

Refer  to  Figure  13  ,  Sheets  1    and  2

Refer  to  Figure  19

Refer  to  Figure  20

Refer  to  Figure  24

Refer  to  Kit  211908A02R08

1    In  the  overhead  panel  20VU,  remove  the  equipment  for

access .

Refer  to  Figure  12

Refer  to  Figure  13  ,  Sheets  1    and  2

a    Remove  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   39

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

FIN  30VU

1        Panel                                                           Item  (127)  Discard

FIN  12LF

1        Plate                          723-4071-03          Item  (125)  Discard

attached  with  :

5        Screw                                                           Item  (126)  Retain

b    Remove  and  retain  the  ELEC  PNL  35VU,  the  HYD/FUEL  PNL

40VU  and  the  OVERHEAD  CTL&IND  PNL  48VU .

c    Remove  the  ENG/APU  fire  panel  (1WD)  (Refer  to  AMM

26-12-12  Page  block  401) .

d    Open  without  disconnect  the  OVERHEAD  C/B  PNL  49VU .

2    Install  the wires  as  shown  on  the wiring  diagram,  Figure  20

,  Sheet 1    and  as  given  in  the  Hook-up  Chart  lines  :

—  1  thru  6  (Refer  to  Figure  24  ,  Sheet  1)  supplied  in  :

Bundle

D9000095220397

3    Attach  the  wires  with  :

400        Tie-Cable                  NSA935401-03

400        Tie-Cable                  NSA935401-04

400        Tie-Cable                  NSA935401-05

400        Tie-Cable                  NSA935401-08

4    With  the  wires,  install  :

Refer  to  Figure  13  ,  Sheet  1

FIN  2432VC-A

1        Connector-Plug        EN3646A61626BZ    Item  128

FIN  2432VC-A1

1        Backshell                  E0080-01-16C        Item  129

5    Install  the  identification  plate  2432VC-A  supplied  in  :

Bundle                        D9000095220397

6    In  the  overhead  panel  20VU,  install  the  equipment .

Refer  to  Figure  12

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   40

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  13  ,  Sheets  1    and  2

a    Install  :

Refer  to  Figure  13  ,  Sheets  1    and  2

FIN  30VU

| 1 | Panel FIN 12LF | D9550003000197 | Item 127 |
| --- | --- | --- | --- |
| 1 | Placard with : | D33110306C00 | Item 125 |
| 5 | Screw |  | Item (126) |

NOTE  :  Screw,  Item  (126)  was  retained  at  removal .

b    Install  the  ELEC  PNL  35VU,  the  HYD/FUEL  PNL  40VU  and  the

OVERHEAD  CTL&IND  PNL  48VU .

c    Install  the  ENG/APU  fire  panel  (1WD)  (Refer  to  AMM

26-12-12  Page  block  401) .

d    Close  the  OVERHEAD  C/B  PNL  49VU .

(h)    In  the  RH  side,  modify  the  routing  between  FR2  and  FR3 .

Refer  to  Figure  15  ,  Sheets  1    and  2

1    Install  as  shown  in  section  B-B  :

1        Mount-Cable              E0343-01

with  :

1        Screw                          NAS1801-3-9

1        Washer                        NAS1149F0332P

2    Install  as  shown  in  section  C-C  :

1        Mount-Cable              E0343-01

with  :

1        Screw                          NAS1801-3-9

1        Washer                        NAS1149F0332P

3    Remove  as  shown  in  section  D-D  :

1        Mount-Cable

Item

Item

Item

Item

Item

Item

Item

116

52

130

116

52

130

(119)  Discard

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   41

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

with  its  hardware .

4    Install  as  shown  in 2        Mount

with  :

1        Spacer

1        Nut

1        Screw

1        Washer

5    Install  as  shown  in

section  D-D  : NSA935504-01

NSA5527-03-08

NSA5050-3

NAS1801-3-24

NAS1149F0332P

view  A  :

Item

Item

Item

Item

Item

119

120

118

121

130

1  M    Conduit                      E0432A06                Item  115

(i)    In  the  LH  side,  modify  the  routing  between  FR2  and  FR3 . Refer  to  Figure  16  ,  Sheets  1    and  2

1    Install  as  shown  in  section  B-B  :

1        Mount-Cable              E0343-01                Item  116

with  :

1        Screw                          NAS1801-3-9          Item  52

1        Washer                        NAS1149F0332P      Item  130

2    Install  as  shown  in  section  C-C  :

1        Mount-Cable              E0343-01                Item  116

with  :

1        Screw                          NAS1801-3-9          Item  52

1        Washer                        NAS1149F0332P      Item  130

3    Remove  as  shown  in  section  D-D  :

1        Mount-Cable                                               Item  (119)  Discard

with  its  hardware .

4    Install  as  shown  in  section  D-D  :

2        Mount                          NSA935504-01        Item  119

with  :

1        Spacer                        NSA5527-03-08      Item  120

1        Nut                              NSA5050-3              Item  118

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   42

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1        Screw                          NAS1801-3-24        Item  121

1        Washer                        NAS1149F0332P      Item  130 5    Install  as  shown  in  view  A  :

1  M    Conduit                      E0432A06                Item  115

(j)    In  the  avionics  compartment,  modify  the  equipment .

Refer  to  Figure  17

1    Modify  the  terminal  block  1102VT .

a    From  the  position  17,  remove  (if  installed)  :

1        Module- False                                                                   Retain

or

1        Clamp-End                                                                         Retain b    At  the  position  17,  install  :

1        Module                        NSA937901MA2011  Item  122

with  :

1        Module                        NSA937901E1

Identification

1        Module                        NSA937901E7

Identification

c    At  the  position  18,  install  as  required  :

1        Module- False

or

1        Clamp-End

NOTE  :  The  false-module  or  end-clamp  were  retained  at

removal .

(k)    Modify  the wiring  between  the  equipment  (FINs  3HE  and  4HE)  and

the  avionics  compartment .

Refer  to  Figure  6  ,  Sheet  5

Refer  to  Figure  15  ,  Sheets  1    and  2

Refer  to  Figure  16  ,  Sheets  1    and  2

Refer  to  Figure  21

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   43

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  25

1    Install  the wires  as  shown  on  the wiring  diagram,  Figure  21

,  Sheet 1    and  as  given  in  the  Hook-up  Chart  lines  :

—  1  thru  13  (Refer  to  Figure  25  ,  Sheet  1)  supplied  in  :

Bundle

D9000095220397

2    Attach  the  wires  with  :

300        Tie-Cable                  NSA935401-03

300        Tie-Cable                  NSA935401-04

300        Tie-Cable                  NSA935401-05

300        Tie-Cable                  NSA935401-08

3    In  the  cockpit,  at  the  end  of  the  wires,  install  :

Refer  to  Figure  6  ,  Sheet  5

FINs  3HE-A  and  4HE-A

2        Connector-Plug        EN3646A61006BN    Item  123

FINs  3HE-A1  and  4HE-A1

2        Backshell                  E0080-02-10C        Item  124

4    Install  the  identification  plates  3HE-A  and  4HE-A  supplied

in  :

Bundle

D9000095220397

5    In  the  RH  and  LH  sides,  connect  the  grounding  points  203VN,

204VN,  205VN,  206VN,  207VN  and  208VN  with  :

6        Screw                          NAS1801-3-8          Item  37

6        Nut                              NSA5050-3              Item  118

6        Washer                        NAS1149F0332P      Item  130

(l)    Modify  the  wiring  in  the  relay  box  103VU .

Refer  to  Figure  22

Refer  to  Figure  26

1    Install  the  wire  as  shown  on  the  wiring  diagram,  Figure  22

,  Sheet  1    and  as  given  in  the  Hook-up  Chart  line   :

—  1  (Refer  to  Figure  26  ,  Sheet  1)  supplied  in  :

Bundle                        D9000095220397

2    Attach  the  wire  with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   44

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

50        Tie-Cable                  NSA935401-03

50        Tie-Cable                  NSA935401-04

50        Tie-Cable                  NSA935401-05

50        Tie-Cable                  NSA935401-08

(m)    Install  the  duct  Item  (90) .

Refer  to  Figure  1  ,  Sheet  1

1    Install  :

1        Duct                                                             Item  (90)    Retain

attach  with  :

Item  (91)    Retain

Item  (92)    Retain

Item  (95)    Retain

Item  (96)    Retain

Item  (93)    Retain

Item  (94)    Retain

1

1

2

1

2

1

Clamp

Clamp

Clamp

Sleeve

Clamp

Sleeve

NOTE  :  The  duct  Item  (90),  the  sleeves  Item  (94)  and  Item

(96)  and  the  clamps  Item  (91),  Item  (92),  Item  (93)

and  Item  (95) .

(n)    In  the  cockpit,  install  the  equipment  removed  for  access .

1    Install  the  captain  and  first  officer  foot  warmers  (2DR1,

2DR2,  2DR3  and  2DR4)  (Refer  to  AMM  25-15- 51  Page  block

401) .

2    Install  the  foot  rest  assembly  (Refer  to  AMM  25-13-14  Page

block  401) .

3    Install  the  captain  and  first  officer  pedals  (Refer  to  AMM

27-21-42  Page  block  401) .

4    Install  the  captain  and  first  officer  seats  (3MS  and  4MS)

(Refer  to  AMM  25-11- 51  Page  block  401) .

(3)    Config .  03

(a)    In  the  cockpit,  remove  the  equipment  for  access .

Refer  to  AMM  25-11- 51

Refer  to  AMM  27-21-42

refer  to  AMM  25-13-14

Refer  to  AMM  25-15- 51

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   45

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1    Remove  the  captain  and  first  officer  seats  (3MS  and  4MS)

(Refer  to  AMM  25-11- 51  Page  block  401) .

2    Remove  the  captain  and  first  officer  pedals  (Refer  to  AMM

27-21-42  Page  block  401) .

3    Remove  the  foot  rest  assembly  (Refer  to  AMM  25-13-14  Page

block  401) .

4    Remove  the  captain  and  first  officer  foot  warmers  (2DR1,

2DR2,  2DR3  and  2DR4)  (Refer  to  AMM  25-15- 51  Page  block

401) .

5    Remove  the  duct  Item  (90)  for  access .

Refer  to  Figure  1  ,  Sheet  1

a    Remove  :

1        Duct                                                             Item  (90)    Retain

attached  with  :

1        Clamp                                                           Item  (91)    Retain

1        Clamp                                                           Item  (92)    Retain

2        Clamp                                                           Item  (95)    Retain

1        Sleeve                                                         Item  (96)    Retain

2        Clamp                                                           Item  (93)    Retain

1        Sleeve                                                         Item  (94)    Retain

(b)    Install  the  tape  adhesive  Item  61  on  the  panel  assy  at  LH

side .

Refer  to  Figure  4  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

(c)    Install  the  tape  adhesive  Item  61  on  the  panel  assy  at  RH

side .

Refer  to  Figure  4  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   46

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(d)    Install  the  heater  3HE  at  LH  side .

Refer  to  Figure  5  ,  Sheet  1

Refer  to  Figure  6  ,  Sheets  1    thru  5

Refer  to  Figure  7  ,  Sheets  1    thru  4

Refer  to  Figure  8  ,  Sheets  1    and  2

Refer  to  Kit  211908A02R08

1    Replace  the  cabin  floor  panel .

Refer  to  Figure  5  ,  Sheet  1

a    Remove  the  cabin  floor  Item  (18) .

—  Disconnect  the  connector  1025VC  for  access .

—  Remove  :

1        Cabin  floor                                               Item  (18)    Retain

attached  with  :

6        Screw                                                           Item  (29)    Retain

b    Install  the  cabin  floor  Item  18.

—  Install  :

1        Panel  -  floor          D5361107200000    Item  18

with  :

6        Screw                                                           Item  (29)

NOTE  :  The  screw  Item  (29)  was  retained  at  removal .

—  Connect  the  connector  1025VC  already  disconnected  for

access .

2    Install  the  ducts  and  the  heater  3HE .

Refer  to  Figure  2  ,  Sheets  1 Refer  to  Figure  6  ,  Sheets  1 Refer  to  Figure  7  ,  Sheets  1 Refer  to  Figure  8  ,  Sheets  1

a    Remove  the  duct  Item  (74) .

and  2

thru  5

thru  4

and  2

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   47

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  6  ,  Sheet  1

—  Remove  :

1        Duct

attached  with  :

2        Clamp

and

1         Base  assy

mounting

1        Outlet  air

attached  with  :

4        Screw

b    Disconnect  the  cable  Item  (80) .

Refer  to  Figure  6  ,  Sheet  1

—  Disconnect  :

1        Cable

attached  with  :

1        Bolt

1        Clamp

1        Screw

1        Washer

1        Nut

Item  (74)    Retain

Item  (73)    Retain

Item  (83)    Retain

Item  (84)    Retain

Item  (85)    Retain

Item  (80)    Retain

Item  (75)    Retain

Item  (77)    Retain

Item  (79)    Retain

Item  (78)    Discard

Item  (76)    Discard

—  Attach  temporarily  the  cable  Item  (80)  on  the

structure  with  :

1        Tie-Cable                  NSA935401-05

c    Install  a  tie-cable  on  the  flap  of  the  air  outlet .

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1        Tie-Cable                  NSA935401-05

NOTE  :  The  tie-cable  is  installed  to  lock  the  flap  of

the  air  outlet  in  close  position .

d    Install  the  tape-adhesive  Item  61  (only  on  the  2342  ) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   48

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@

Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

e    Install  the  brackets  Item  54,

the  structure .

Refer  to  Figure  6  ,  Sheets  2

Refer  to  Figure  7  ,  Sheets  1

Refer  to  Figure  8  ,  Sheets  1

—  Install  the  bracket  Item  54

55,  20,  22,  23,  7  and  8  on

thru  5

thru  4

and  2

on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  54  PN

D9251587500000  on  the  structure  as  shown  on  Figure

6,  Sheet  2.

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587500000    Item  54

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  55  on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  55  PN

D9251587600000  on  the  structure  as  shown  on  Figure

6,  Sheet  2  on  detail  C .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587600000    Item  55

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   49

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  Install  the  bracket  Item  20,  Item  22,  Item  23  on  the

cabin  floor .

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Temporarily  put  in  position  the  brackets  Item  20  PN

D5392514820000,  Item  22  PN  D5392514920000,  Item  23

PN  D5391828200000  on  the  structure  as  shown  on

Figure  6,  Sheet  3  on  detail  D .

.  Drill  and  deburr  eight  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  3  detail  D .

1

3

1

3

1

2

1

.  Install  :

Bracket                      D5392514820000    Item  20

with  :

Rivet                          ASNA2050DCJ3215  Item  56

and

Bracket                      D5392514920000    Item  22

with  :

Rivet                          ASNA2050DCJ3215  Item  56

and

Bracket                      D5391828200000    Item  23

with  :

Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  23  between  FR2  and  FR3  on  the

under  floor .

Refer  to  Figure  7  ,  Sheets  1    and  4

.  Temporarily  put  in  position  the  bracket  Item  23  PN

D5391828200000  on  the  structure  as  shown  on  Figure

6,  Sheet  4  on  detail  F .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  4.

.  Install  :

Bracket                      D5391828200000    Item  23

with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    50

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2        Rivet                          ASNA2050DCJ3215  Item  56

3

6

6

6

1

2

2

1

2

—  Install  the  brackets  Item  7,  Item  8  and  their  related

shims  on  the  floor  panel .

Refer  to  Figure  6  ,  Sheets  2    thru  5

.  Install  :

Holder                        D2122873020000    Item  7

Shim                            D2122875620000    Item  11

with  :

| Screw Washer and Support assy with : Screw Washer — Install the | NAS1801-3-8 Item 37 ASNA2397-10L Item 39 D2122873100000 Item 8 NAS1801-3-6 Item 40 ASNA2397-10L Item 39 nut Item 81 on the cabin floor . |
| --- | --- |

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Drill  and  deburr  one  hole  on  the  cabin  floor  to  the

dimensions  given  in  Figure  6,  Sheet  3,  detail  D .

.  Temporarily  put  in  position  the  nut  Item  81  PN

MS21071L3  on  the  cabin  floor  as  shown  on  Figure  6,

Sheet  3,  detail  D .

.  Drill  and  deburr  two  holes  on  the  cabin  floor  to  the

dimensions  given  in  Figure  6,  Sheet  3,  detail  D .

.  Install  :

Nut                              MS21071L3              Item  81

with  :

Rivet-Solid              ASNA2051DCJ2412  Item  82

—  As  shown  on  Figure  5,  Sheet  4,  detail  G,  bond  the

placard  HEATER  4HE  supplied  in

Placard  set              D9100095212195

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    51

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

f    Install  the  grounding  points  and  the  placards  203VN,

205VN,  207VN .

Refer  to  Figure  8  ,  Sheets  1    and  2

—  Drill  three  holes  on  the  structure  to  a  diameter  of

5 .2mm  (0 .204in .)  as  shown  on  Figure  8,  Sheet  1.

—  On  the  structure,  do  a  stripping  to  the  dimension

given  in  Refer  to  Figure  7,  Sheet  1.

—  As  shown  on  Figure  7,  Sheet  2,  detail  A,  bond  the

placards  203VN,  205VN,  207VN,  supplied  in  :

Placard  set              D9100095212195

g

Install  the  ducts  and  the  heater  3HE .

Refer  to  Figure  6  ,  Sheets  2    thru  5

—  Install  :

| 1 | Hose with : | ABS1114B020D350 | Item | 51 |
| --- | --- | --- | --- | --- |
| 1 | Clamp and | ABS0901-035 | Item | 50 |
| 1 | Tee - Helded | D2122872600000 | Item | 1 |
| 1 | Sleeve with : | ASNA3323-203 | Item | 13 |
| 3 | Clamp and | ABS0901-035 | Item | 50 |
| 1 | Heater | RC0200A00 | Item | 53 |
| 1 | Valve 3 ways | F0003078400000 | Item | 14 |
| 1 | Valve with : | D2122872700000 | Item | 2 |
| 4 | Screw | NAS1801-3-9 | Item | 52 |
| 4 | Screw | NAS1801-3-7 | Item | 36 |
| 8 | Washer | ASNA2397-10L | Item | 39 |
| 8 | Nut and | MS21042-3 | Item | 38 |
| 1 | Hose | ABS1114B028D640 | Item | 35 |

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    52

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

with  :

2        Clamp                          ABS0901-045          Item  42

NOTE  :  Attach  the  hose  Item  35  on  the  existing  duct

with  Tape  Hook  and  Loop  ABS1133A025    and  Self

Adhesive  ABS1133D025  .

and

1        Outlet  -  blowing    D2122872800000    Item  4

with  :

3        Clamp                          ABS0901-050          Item  45

and

2        Cap  by  pass              D2122872900000    Item  6

with  :

2        Clamp                          ABS0901-050          Item  45

and

1        Hose                            ABS1114B028D350  Item  34

with  :

2        Clamp                          ABS0901-045          Item  42

1        Clamp                          NSA5516CA54NJ      Item  41

2        Screw                          NAS1801-3-7          Item  36

2        Washer                        ASNA2397-10L        Item  39

h    Install  the  cable  Item  10.

Refer  to  Figure  6  ,  Sheets  2    and  Sheet  3

—  Install  :

1        Cable                          D0003005501000    Item  10

with  :

2

1

2

1

1

5

4

1

Screw

Screw

Screw

Bolt

Spacer

Washer

Clamp

Clamp

NAS1801-3-7

NAS1801-3-12

NAS1801-3-8

F2121008920000

NSA5527-03-05

ASNA2397-10L

NSA5516A06NJ

NSA5516A07NJ

Item

Item

Item

Item

Item

Item

Item

Item

36

49

37

12

46

39

43

44

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    53

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2

1

Washer

Nut

ASNA2397-416

MS21042-4

Item  47

Item  48

(e)    Install  the  heater  4HE  at  RH  side .

Refer  to  Figure  5  ,  Sheet  1

Refer  to  Figure  6  ,  Sheets  1  ,  thru  5

Refer  to  Figure  7  ,  Sheets  1    thru  4

Refer  to  Figure  9  ,  Sheets  1    and  2

Refer  to  Kit  211908A02R08

Refer  to  Kit  211908A03R03

1    Replace  the  cabin  floors  panel .

Refer  to  Figure  5  ,  Sheet  1

a    Remove  the  cabin  floor  Item  (19) .

—  Remove  :

1        Cabin  floor                                               Item  (19)    Retain

attached  with  :

6        Screw                                                           Item  (29)    Retain

b    Install  the  cabin  floor  Item  19.

—  Install  :

1        Panel  -  floor          D5361107200100    Item  19

with  :

6        Screw                                                           Item  (29)

NOTE  :  The  screw  Item  (29)  was  retained  at  removal .

2    Install  the  ducts  and  the  heater  4HE .

Refer  to  Figure  2  ,  Sheets  1 Refer  to  Figure  6  ,  Sheets  1 Refer  to  Figure  7  ,  Sheets  1 Refer  to  Figure  9  ,  Sheets  1

a    Remove  the  duct  Item  (74) .

and  2

thru  5

thru  4

and  2

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    54

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  6  ,  Sheet  1

—  Remove  :

1        Duct

attached  with  :

2        Clamp

and

1         Base  assy

mounting

1        Outlet  air

attached  with  :

4        Screw

b    Disconnect  the  cable  Item  (80) .

Refer  to  Figure  6  ,  Sheet  1

—  Disconnect  :

1        Cable

attached  with  :

1        Bolt

1        Clamp

1        Screw

1        Washer

1        Nut

Item  (74)    Retain

Item  (73)    Retain

Item  (83)    Retain

Item  (84)    Retain

Item  (85)    Retain

Item  (80)    Retain

Item  (75)    Retain

Item  (77)    Retain

Item  (79)    Retain

Item  (78)    Discard

Item  (76)    Discard

—  Attach  temporarily  the  cable  Item  (80)  on  the

structure  with  :

1        Tie-Cable                  NSA935401-05

c    Install  a  tie-cable  on  the  flap  of  the  air  outlet .

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1        Tie-Cable                  NSA935401-05

NOTE  :  The  tie-cable  is  installed  to  lock  the  flap  of

the  air  outlet  in  close  position .

d    Install  the  tape-adhesive  Item  61  (only  on  the  2342  ) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    55

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@

Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  2  ,  Sheets  1    and  2

—  Install  :

1  M    Tape-Adhesive          ABS5006-2G            Item  61

NOTE  :  The  tape-adhesive  is  installed  to  close  the  air

outlet .

e    Install  the  brackets  Item  54,

the  structure .

Refer  to  Figure  6  ,  Sheets  1

Refer  to  Figure  7  ,  Sheets  1

Refer  to  Figure  9  ,  Sheets  1

—  Install  the  bracket  Item  54

55,  21,  22,  23,  7  and  9  on

thru  5

thru  4

and  2

on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  54  PN

D9251587500000  on  the  structure  as  shown  on  Figure

6,  Sheet  2  on  detail  B .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587500000    Item  54

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  55  on  the  structure .

Refer  to  Figure  7  ,  Sheets  1    and  2

.  Temporarily  put  in  position  the  bracket  Item  55  PN

D9251587600000  on  the  structure  as  shown  on  Figure

6,  Sheet  2  on  detail  C .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  2.

.  Install  :

1        Bracket                      D9251587600000    Item  55

with  :

2        Rivet                          ASNA2050DCJ3215  Item  56

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    56

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  Install  the  bracket  Item  21,  Item  22,  Item  23  on  the

cabin  floor .

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Temporarily  put  in  position  the  brackets  Item  21  PN

D5392514820100,  Item  22  PN  D5392514920000,  Item  23

PN  D5391828200000  on  the  structure  as  shown  on

Figure  6,  Sheet  3  on  detail  D .

.  Drill  and  deburr  eight  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  3.

1

3

1

3

1

2

1

.  Install  :

Bracket                      D5392514820100    Item  21

with  :

Rivet                          ASNA2050DCJ3215  Item  56

and

Bracket                      D5392514920000    Item  22

with  :

Rivet                          ASNA2050DCJ3215  Item  56

and

Bracket                      D5391828200000    Item  23

with  :

Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  bracket  Item  23  between  FR2  and  FR3  on  the

under  floor .

Refer  to  Figure  7  ,  Sheets  1    and  4

.  Temporarily  put  in  position  the  bracket  Item  23  PN

D5391828200000  on  the  structure  as  shown  on  Figure

6,  Sheet  4  on  detail  F .

.  Drill  and  deburr  two  holes  on  the  structure  to  the

dimensions  given  in  Figure  6,  Sheet  4.

.  Install  :

Bracket                      D5391828200000    Item  23

with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    57

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

2        Rivet                          ASNA2050DCJ3215  Item  56

—  Install  the  brackets  Item  7,  Item  9  and  their  related

shims  on  the  floor  panel .

Refer  to  Figure  6  ,  Sheets  2    thru  5

2

1

3

3

6

6

1

1

2

2

.  Install  :

Holder

Holder

Shim

Shim

with  :

Screw

Washer

and

Support  assy

Shim

with  :

Screw

Washer

D2122873020000

D2122878020000

D2122875620000

D2122875620200

NAS1801-3-15

ASNA2397-10L

D2122873100100

D2122875620400

NAS1801-3-13

ASNA2397-10L

Item

Item

Item

Item

Item

Item

Item

Item

Item

Item

7

131

11

132

135

39

9

133

134

39

—  Install  the  nut  Item  81  on  the  cabin  floor .

Refer  to  Figure  7  ,  Sheets  1    and  3

.  Drill  and  deburr  one  hole  on  the  cabin  floor  to  the

dimensions  given  in  Figure  6,  Sheet  3,  detail  D .

.  Temporarily  put  in  position  the  nut  Item  81  PN

MS21071L3  on  the  cabin  floor  as  shown  on  Figure  6,

Sheet  3,  detail  D .

.  Drill  and  deburr  two  holes  on  the  cabin  floor  to  the

dimensions  given  in  Figure  6,  Sheet  3,  detail  D .

.  Install  :

1        Nut                              MS21071L3              Item  81

with  :

2        Rivet-Solid              ASNA2051DCJ2412  Item  82

—  As  shown  on  Figure  5,  Sheet  4,  detail  G,  bond  the

placard  HEATER  4HE  supplied  in

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    58

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Placard  set              D9100095212195

f    Install  the  grounding  points  and  the  placards  204VN,

206VN,  208VN .

Refer  to  Figure  9  ,  Sheets  1    and  2

—  Drill  three  holes  on  the  structure  to  a  diameter  of

5 .2mm  (0 .204in .)  as  shown  on  Figure  8,  Sheet  1.

—  On  the  structure,  do  a  stripping  to  the  dimension

given  in  Figure  8,  Sheet  1.

—  As  shown  on  Figure  8,  Sheet  2,  detail  A,  bond  the

placards  204VN,  206VN,  208VN,  supplied  in  :

Placard  set              D9100095212195

g

Install  the  ducts  and  the  heater  4HE .

Refer  to  Figure  6  ,  Sheets  2    thru  5

—  Install  :

1        Hose                            ABS1114B020D350  Item  51

with  :

1        Clamp                          ABS0901-035          Item  50

and

1        Tee  -  Helded            D2122872600000    Item  1

1        Sleeve                        ASNA3323-203        Item  13

with  :

3        Clamp                          ABS0901-035          Item  50

and

1        Heater                        RC0200A00              Item  53

1        Valve  3  ways            F0003078400100    Item  15

1        Valve                          D2122872700100    Item  3

with  :

4        Screw                          NAS1801-3-9          Item  52

4        Screw                          NAS1801-3-7          Item  36

8        Washer                        ASNA2397-10L        Item  39

8        Nut                              MS21042-3              Item  38

and

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :    59

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1        Hose                            ABS1114B028D640  Item  35

with  :

2        Clamp                          ABS0901-045          Item  42

NOTE  :  Attach  the  hose  Item  35  on  the  existing  duct

with  Tape  Hook  and  Loop  ABS1133A025    and  Self

Adhesive  ABS1133D025  .

and

1        Outlet  -  blowing    D2122872800100    Item  5

with  :

3        Clamp                          ABS0901-050          Item  45

and

2        Cap  by  pass              D2122872900000    Item  6

with  :

2        Clamp                          ABS0901-050          Item  45

and

1        Hose                            ABS1114B028D350  Item  34

with  :

2        Clamp                          ABS0901-045          Item  42

1        Clamp                          NSA5516CA54NJ      Item  41

2        Screw                          NAS1801-3-7          Item  36

2        Washer                        ASNA2397-10L        Item  39

h    Install  the  cable  Item  10.

Refer  to  Figure  6  ,  Sheets  2    and  Sheet  3

—  Install  :

1        Cable                          D0003005501000    Item  10

with  :

2

1

2

1

1

5

Screw

Screw

Screw

Bolt

Spacer

Washer

NAS1801-3-7

NAS1801-3-12

NAS1801-3-8

F2121008920000

NSA5527-03-05

ASNA2397-10L

Item

Item

Item

Item

Item

Item

36

49

37

12

46

39

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   60

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

4        Clamp

1        Clamp

2        Washer

1        Nut

(f)    Modify  the  equipment

Refer  to  Figure  11  ,

Refer  to  Figure  18

Refer  to  Figure  23

NSA5516A06NJ

NSA5516A07NJ

ASNA2397-416

MS21042-4

and  the  wiring  in

Sheets  1    and  2

Item  43

Item  44

Item  47

Item  48

the  rear  panel  120VU .

Refer  to  Kit  211908A05R01

1    Install  the  circuit  breaker  1HE  and  the  related  placard

44LM .

Refer  to  Figure  11  ,  Sheets  1    and  2

a    From  the  rear  C/B  panel  122VU,  remove  :

FIN  44LM

1        Placard                      D11311478A00        Item  (107)  Discard

attached  with  :

4        Button  Rivet                                             Item  (108)  Discard

Plastic

b    On  the  rear  C/B  panel  122VU,  install  :

FIN  44LM

1        Placard                      D11311898B00        Item  107

with  :

4        Button  Rivet            ASNA2080G04          Item  108

Plastic

c    In  the  rear  C/B  panel  122VU,  install  at  the  location V23

:

FIN  1HE

1        Circuit  Breaker      NSA931320-050      Item  100

equipped  with  :

2

2

Screw

Washer-Locking

E0736N08-01

ASNA2553-0401

Item  109

Item  110

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   61

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

d    Near  the  circuit  breaker,  bond  the  placard  1HE,  supplied

in  :

Placard  set              D9100095212195

2    Modify  the  wiring  in  the  rear  panel  120VU .

Refer  to  Figure  11  ,  Sheets  1    and  2

Refer  to  Figure  18

Refer  to  Figure  23

a    Install  the wires  as  shown  on  the wiring  diagram,  Figure

18  ,  Sheet  1    and  as  given  in  the  Hook-up  Chart  lines  :

—  1  and  2  (Refer  to  Figure  23    Sheet  1)  supplied  in  :

Bundle                        D9000095220397

b    Attach  the  wires  with  :

150        Tie-Cable                  NSA935401-03

150        Tie-Cable                  NSA935401-04

150        Tie-Cable                  NSA935401-05

150        Tie-Cable                  NSA935401-08

(g)    Modify  the  equipment  and  the  wiring  between  the  cockpit  and

the  avionics  compartment .

Refer  to  Figure  10

Refer  to  Figure  12

Refer  to  Figure  14  ,  Sheets  1    and  2

Refer  to  Figure  19

Refer  to  Figure  20

Refer  to  Figure  24

Refer  to  Kit  211908A05R01

1    In  the  overhead  panel  20VU,  remove  the  equipment  for

access .

Refer  to  Figure  12

Refer  to  Figure  14  ,  Sheets  1    and  2

a    Remove  :

FIN  30VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   62

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1        Panel                                                           Item  (127)  Discard

FIN  12LF

1        Plate-Integrally    723-4207-01          Item  (125)  Discard

Lighted

attached  with  :

5        Screw                                                           Item  (126)  Retain

b    Remove  and  retain  the  ELEC  PNL  35VU,  the  HYD/FUEL  PNL

40VU  and  the  OVERHEAD  CTL&IND  PNL  48VU .

c    Remove  the  ENG/APU  fire  panel  (1WD)  (Refer  to  AMM

26-12-12  Page  block  401) .

d    Open  without  disconnect  the  OVERHEAD  C/B  PNL  49VU .

2    Install  the wires  as  shown  on  the wiring  diagram,  Figure  20

,  Sheets 1    and  as  given  in  the  Hook-up  Chart  lines  :

—  1  thru  6  (Refer  to  Figure  24  ,  Sheet  1)  supplied  in  :

Bundle                        D9000095220397

3    Attach  the  wires  with  :

400        Tie-Cable                  NSA935401-03

400        Tie-Cable                  NSA935401-04

400        Tie-Cable                  NSA935401-05

400        Tie-Cable                  NSA935401-08

4    With  the  wires,  install  :

Refer  to  Figure  14  ,  Sheets  1    and  2

FIN  2432VC-A

1        Connector-Plug        EN3646A61626BZ    Item  128

FIN  2432VC-A1

1        Backshell                  E0080-01-16C        Item  129

5    Install  the  identification  plate  2432VC-A  supplied  in  : Bundle                        D9000095220397

6    In  the  overhead  panel  20VU,  install  the  equipment .

Refer  to  Figure  12

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   63

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@

Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  14  ,  Sheets  1    and  2

a    Install  :

Refer  to  Figure  14  ,  Sheets  1    and  2

FIN  30VU

1        Panel                          D9550003000295    Item  127

FIN  12LF

1        Placard                      D33110305B00        Item  125

with  :

5        Screw                                                           Item  (126)

NOTE  :  Screw,  Item  (126)  was  retained  at  removal .

b    Install  the  ELEC  PNL  35VU,  the  HYD/FUEL  PNL  40VU  and  the

OVERHEAD  CTL&IND  PNL  48VU .

c    Install  the  ENG/APU  fire  panel  (1WD)  (Refer  to  AMM

26-12-12  Page  block  401) .

d    Close  the  OVERHEAD  C/B  PNL  49VU .

(h)    In  the  RH  side,  modify  the  routing  between  FR2  and  FR3 .

Refer  to  Figure  15  ,  Sheets  1    and  2

1    Install  as  shown  in  section  B-B  :

1        Mount-Cable              E0343-01

with  :

1        Screw                          NAS1801-3-9

1        Washer                        NAS1149F0332P

2    Install  as  shown  in  section  C-C  :

1        Mount-Cable              E0343-01

with  :

1        Screw                          NAS1801-3-9

1        Washer                        NAS1149F0332P

3    Remove  as  shown  in  section  D-D  :

1        Mount-Cable

Item

Item

Item

Item

Item

Item

Item

116

52

130

116

52

130

(119)  Discard

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   64

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

with  its  hardware .

4    Install  as  shown  in 2        Mount

with  :

1        Spacer

1        Nut

1        Screw

1        Washer

5    Install  as  shown  in

section  D-D  : NSA935504-01

NSA5527-03-08

NSA5050-3

NAS1801-3-24

NAS1149F0332P

view  A  :

Item

Item

Item

Item

Item

119

120

118

121

130

1  M    Conduit                      E0432A06                Item  115

(i)    In  the  LH  side,  modify  the  routing  between  FR2  and  FR3 . Refer  to  Figure  16  ,  Sheets  1    and  2

1    Install  as  shown  in  section  B-B  :

1        Mount-Cable              E0343-01                Item  116

with  :

1        Screw                          NAS1801-3-9          Item  52

1        Washer                        NAS1149F0332P      Item  130

2    Install  as  shown  in  section  C-C  :

1        Mount-Cable              E0343-01                Item  116

with  :

1        Screw                          NAS1801-3-9          Item  52

1        Washer                        NAS1149F0332P      Item  130

3    Remove  as  shown  in  section  D-D  :

1        Mount-Cable                                               Item  (119)  Discard

with  its  hardware .

4    Install  as  shown  in  section  D-D  :

2        Mount                          NSA935504-01        Item  119

with  :

1        Spacer                        NSA5527-03-08      Item  120

1        Nut                              NSA5050-3              Item  118

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   65

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1        Screw                          NAS1801-3-24        Item  121

1        Washer                        NAS1149F0332P      Item  130 5    Install  as  shown  in  view  A  :

1  M    Conduit                      E0432A06                Item  115

(j)    In  the  avionics  compartment,  modify  the  equipment .

Refer  to  Figure  17

1    Modify  the  terminal  block  1102VT .

a    From  the  position  17,  remove  (if  installed)  :

1        Module- False                                                                   Retain

or

1        Clamp-End                                                                         Retain b    At  the  position  17,  install  :

1        Module                        NSA937901MA2011  Item  122

with  :

1        Module                        NSA937901E1

Identification

1        Module                        NSA937901E7

Identification

c    At  the  position  18,  install  as  required  :

1        Module- False

or

1        Clamp-End

NOTE  :  The  false-module  or  end-clamp  were  retained  at

removal .

(k)    Modify  the wiring  between  the  equipment  (FINs  3HE  and  4HE)  and

the  avionics  compartment .

Refer  to  Figure  6  ,  Sheet  5

Refer  to  Figure  15  ,  Sheets  1    and  2

Refer  to  Figure  16  ,  Sheets  1    and  2

Refer  to  Figure  21

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   66

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Refer  to  Figure  25

1    Install  the wires  as  shown  on  the wiring  diagram,  Figure  21

,  Sheets 1    and  as  given  in  the  Hook-up  Chart  lines  :

—  1  thru  13  (Refer  to  Figure  25  ,  Sheet  1)  supplied  in  :

Bundle                        D9000095220397

2    Attach  the  wires  with  :

300        Tie-Cable                  NSA935401-03

300        Tie-Cable                  NSA935401-04

300        Tie-Cable                  NSA935401-05

300        Tie-Cable                  NSA935401-08

3    In  the  cockpit,  at  the  end  of  the  wires,  install  :

Refer  to  Figure  6  ,  Sheet  5

FINs  3HE-A  and  4HE-A

2        Connector-Plug        EN3646A61006BN    Item  123

FINs  3HE-A1  and  4HE-A1

2        Backshell                  E0080-02-10C        Item  124

4    Install  the  identification  plates  3HE-A  and  4HE-A  supplied

in  :

Bundle                        D9000095220397

5    In  the  RH  and  LH  sides,  connect  the  grounding  points  203VN,

204VN,  205VN,  206VN,  207VN  and  208VN  with  :

6        Screw                          NAS1801-3-8          Item  37

6        Nut                              NSA5050-3              Item  118

6        Washer                        NAS1149F0332P      Item  130

(l)    Modify  the  wiring  in  the  relay  box  103VU .

Refer  to  Figure  22

Refer  to  Figure  26

1    Install  the  wires  as  shown  on  the  wiring  diagram,  Figure  22

,  Sheets  1    and  as  given  in  the  Hook-up  Chart  line   :

—  1  (Refer  to  Figure  26  ,  Sheet  1)  supplied  in  :

Bundle                        D9000095220397

2    Attach  the  wire  with  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   67

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

50        Tie-Cable                  NSA935401-03

50        Tie-Cable                  NSA935401-04

50        Tie-Cable                  NSA935401-05

50        Tie-Cable                  NSA935401-08

(m)    In  the  cockpit,  install  the  equipment  removed  for  access .

1    Install  the  captain  and  first  officer  foot  warmers  (2DR1,

2DR2,  2DR3  and  2DR4)  (Refer  to  AMM  25-15- 51  Page  block

401) .

2    Install  the  foot  rest  assembly  (Refer  to  AMM  25-13-14  Page

block  401) .

3    Install  the  captain  and  first  officer  pedals  (Refer  to  AMM

27-21-42  Page  block  401) .

4    Install  the  captain  and  first  officer  seats  (3MS  and  4MS)

(Refer  to  AMM  25-11- 51  Page  block  401) .

(n)    Install  the  duct  Item  (90) .

Refer  to  Figure  1  ,  Sheet  1

1    Install  :

1        Duct                                                             Item  (90)    Retain

attach  with  :

Item  (91)    Retain

Item  (92)    Retain

Item  (95)    Retain

Item  (96)    Retain

Item  (93)    Retain

Item  (94)    Retain

1

1

2

1

2

1

Clamp

Clamp

Clamp

Sleeve

Clamp

Sleeve

NOTE  :  The  duct  Item  (90),

(96)  and  the  clamps

and  Item  (95) .

the  sleeves  Item  (94)  and  Item

Item  (91),  Item  (92),  Item  (93)

## C .    TEST

(1)    Config .  01

(a)    Remove  the  safety  clips  and  tags  and  close  these  circuit

breakers  :

LOCATION

PANEL

FIN

DESIGNATION

49VU

122VU

1LE

2LE

08  H

04  Z

LIGHTING/EMER  LT/CKPT/DOME

LIGHTING/FLOOD/CTR  INST/PNL

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   68

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

LOCATION

PANEL

FIN

DESIGNATION

122VU

121VU

04  Y

15  K

4LF

5TV

LIGHTING/INSTL  LT/MAIN  INST/PNL

AND/PED

AIDS  &  RCDR/FDIMU

(b)    Do  the  tests  after  removal/installation  of  the  MCDU  (3CA1

,3CA2  and  3CA3)  (Refer  to  AMM  22-82-12  Page  block  401) .

(c)    Do  the  tests  after  removal/installation  of  the  RMP  (1RG1,  1RG2

and  1RG3)  (Refer  to  AMM  23-13-13  Page  block  401) .

(d)    Do  the  tests  after  removal/installation  of  the  ACP  (2RN1,

2RN2,  2RN3,  2RN4  and  2RN5)  (Refer  to  AMM  23- 51-12  Page  block

401) .

(e)    Do  the  tests  after  removal/installation  of  the  Weather  Radar

control  unit  (3SQ)  (Refer  to  AMM  34-41-12  Page  block  401) .

(f)    Do  the  tests  after  removal/installation  of  the  ATC/TCAS

control  unit  (3SH)  (Refer  to  AMM  34- 52-12  Page  block  401) .

(g)    For  the  panel  111VU,  do  an  operational  test  of  the  general

illumination  (Refer  to  AMM  33-12-00  Page  block  501),

(h)    Do  an  operational  test  of  the  instrument  and  panel  integral

lighting  (Refer  to  AMM  33-13-00  Page  block  501)

(i)    Do  an  operational  test  of  the  Cockpit  Door  Lock  System  (CDLS)

(Refer  to  AMM  52- 51-00  Page  block  501) .

(2)    Config .  02  thru  Config .  03 (a)    Job  set-up  :

1    Re-connect  the  battery  :

—  For  job  set-up,  refer  to  the  removal/installation  of  the

batteries  (Refer  to  AMM  24- 38- 51  Page  block  401) .

—  Remove  the  blanking  cap  from  the  electrical  connectors .

—  Make  sure  that  the  electrical  connectors  are  clean  and  in

correct  condition .

—  Connect  the  connector  of  the  batteries .

—  Turn  the  knurled  nut  a  quarter  turn .

2    Remove  all  the  warning  notices .

3    Restore  the  systems  and  the  aircraft  to  the  normal

operating  condition .

(b)    Preliminary  test

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   69

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1    Initial  conditions  :

a    On  each  connectors  that  you  disconnected,  do  a  visual

check  as  defined  by  ESPM  20- 52-10,  to  make  sure  that  :

—  The  label  of  the  plug  is  the  same  as  the  label  on  the

receptacle .

—  The  plug  is  correctly  locked .

2    Preliminary  test  :

NOTE  :  If  the  aircraft  is  operated  in  CAT  3  conditions,  you

must  also  do  this  test  :  land  CAT  3  capability  test

(Refer  to  AMM  22-97-00  Page  block  501) .

a    Do  an  operational  test  of  the  instrument  and  panel

integral  lighting  (Refer  to  AMM  33-13-00  Page  block

501) .

b    Do  an  operational  test  of  the  annunciator  light  test

system  in  the  cockpit  (Refer  to  AMM  33-14-00  Page  block

501) .

c    Do  an  operational  test  of  the  avionics  equipment

ventilation  system  (Refer  to  AMM  21-26-00  Page  block

501) .

d    Do  an  operational  test  of  the  Automatic  Flight  System

(AFS)  (Refer  to  AMM  22-96-00  Page  block  501) .

e    Do  an  operational  test  of  the  Electronic  Instrument

System  (EIS)  (Refer  to  AMM  31-60-00  Page  block  501) .

f    Do  an  operational  test  of  the  Electronic  Centralized

Aircraft  Monitoring  (ECAM)  (Refer  to  AMM  31- 50-00  Page

block  501) .

g    Do  the  ground  scanning  of  the  central  warning  system

(Refer  to  AMM  31- 50-00  Page  block  501) .

(c)    Tests  after  disconnection/connection  of  the  plug  1180VC  :

1    Do  an  operational  test  of  the  Ground  Power  Control  Unit

(GPCU)  with  the  Centralized  Fault  Display  System  (CFDS)

(Refer  to  AMM  24-41-00  Page  block  501) .

(d)    Tests  after  disconnection/connection  of  the  plug  1025VC  :

1    Do  an  operational  test  of  the  Cockpit  Voice  Recorder  (CVR)

and  of  the  CVR  channel  recording  (Refer  to  AMM  23-71-00

Page  block  501) .

(e)    Tests  after  disconnection/connection  of  the  plug  1200VC  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   70

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1    Do  an  operational  test  of  the  parking  brake  system  (Refer

to  AMM  32-45-00  Page  block  501) .

(f)    Tests  after  disconnection/connection  of  the  plug  2252VC  :

1    Do  an  operational  test  of  the  engine  fire  and  overheat

detection  (Refer  to  AMM  26-12-00  Page  block  501) .

(g)    Test  after  removal/installation  of  the  equipment  :

1    Do  the  test  after  removal/installation  of  the  captain  and

first  officer  seats  (Refer  to  AMM  25-11- 51  Page  block  401) .

2    Do  the  test  after  removal/installation  of  the  captain  and

first  officer  pedals  (Refer  to  AMM  27-21-42  Page  block

401) .

3    Do  the  test  after  removal/installation  of  the  captain  and

first  officer  foot warmer  (Refer  to AMM  25-15- 51  Page  block

401) .

(h)    Test  after  removal/installation  of  the  panels  :

1    Do  the  test  after  removal/installation  of  the  panel  35VU

(Refer  to  AMM  31-10-00  Page  block  201) .

2    Do  the  test  after  removal/installation  of  the  panel  40VU

(Refer  to  AMM  31-10-00  Page  block  201) .

3    Do  the  test  after  removal/installation  of  the  panel  48VU

(Refer  to  AMM  31-10-00  Page  block  201) .

4    Do  the  test  after  removal/installation  of  the  ENG/APU  fire

panel  (Refer  to  AMM  26-12-12  Page  block  401) .

(i)    Test  after  removal/installation  of  the  30VU  panel  :

1    Do  an  operational  test  of  the  flow  control  and  indicating

system  (Refer  to  AMM  21- 51-00  Page  block  501) .

2    Do  an  operational  test  of  the  flow  control  valves  closing

(Refer  to  AMM  21- 51-00  Page  block  501) .

3    Do  an  operational  test  of  the  ram-air  inlet  (Refer  to  AMM

21- 55-00  Page  block  501) .

4    Do  a  functional  test  of  the  cockpit  and  cabin  temperature

control  without  Centralized  Fault  Display  System  (CFDS)  /  Multipurpose  Control  and  Display  Unit  (MCDU)  (Refer  to  AMM 21-63-00  Page  block  501) .

5    Do  an  operational  test  of  the  bleed  air  system  (Refer  to

AMM  36-11-00  Page  block  501) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   71

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

6    Do  a  functional  test  of  the  foot  air  outlet  heater  (refer

to  paragraph  C .  (2) .  (h))

(j)    Do  a  voltage  check  at  pin  2  of  each  following  circuit  breakers

to  make  sure  that  there  is  115VAC  :

—  122VU  :  1HE  and  1LC2 .

(k)    Do  an  operational  test  of  the  Circuit  Breaker  Monitoring

System  (CBMS)  (Refer  to  AMM  31- 50-00  Page  block  501) .

(l)    Functional  test  of  the  foot  air  outlet  heater  : 1    Job  set-up  :

a    Energize  the  aircraft  electrical  circuit  (Refer  to  AMM

24-41-00  Page  block  201) .

b    Do  the  EIS  start  procedure  (System  Display  (SD)  Display

Unit  (DU)  only)  (Refer  to  AMM  31-60-00  Page  block  201) .

c    Make  sure  that  this  circuit  breaker  is  closed  :

PANEL

DESIGNATION                                               FIN                  LOCATION

122VU            CKPT  FOOT  HTR/SPLY                                 1HE                  V23

d    Start  the  APU  (Refer  to  AMM  49-00-00  Page  block  501) . e    On  the  AIR  COND  PNL  30VU  :

—  Push  the  PACK  1  and  PACK  2  pushbutton  switches  (OFF

and  FAULT  legends  off),

—  Make  sure  that  the  PACK  FLOW  selector  is  at  NORM,

—  Make  sure  that  the  COCKPIT  and  the  CABIN  temperature

selectors  are  in  the  12  o'clock  position .

—  Make  sure  that  the  cabin  fans  operate .

f    Make  sure  that  all  the  air  outlets  in  the  cockpit  are

open .

g    Make  sure  that  the  cockpit  door  is  closed .

2    Procedure  :

NOTE  :  For  this  test,  the  cockpit  ambient  temperature  must

be  between  20  degrees  C  (68  degrees  F)  and  28

degrees  C  (82 .40  degrees  F) .

NOTE  :  The  temperature  is  shown  on  the AIR  COND  page  of  the

SD .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   72

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

NOTE  :  Use  a  portable  thermometer  to  measure  the

temperature  at  the  individual  air  outlet .

a    Check  of  RH  and  LH  side  valve  3  ways  closing/opening

actuation  :

—  Turn  the  CAPT  and  F/O  air  outlet  actuator  to  closed

position  :

.  On  CAPT  and  F/O  foot  air  outlet,  check  there  is  no

air  flow .

—  Turn  the  CAPT  and  F/O  air  outlet  actuator  to  opened

position  :

.  On  CAPT  and  F/O  foot  air  outlet,  check  there  is  a

constant  air  flow .

b    When  the  blown  air  temperature  is  stable  :

—  On  the AIR  COND  PNL  30VU,  set  the  HEATER  CAPT  selector

switch  to  LO .

c    Measure  the  blown  air  temperature  at  the  left  foot

air-outlet .

NOTE  :  Measure  the  temperature  at  the  outlet .

NOTE  :  Make  sure  that  the  blown-air  stable  temperature

is  12 . 5  degrees  C  (54 . 5  degrees  F)  (+/-  7  degrees

C)  higher  than  the  temperature  shown  on  the  AIR

COND  page  of  the  SD .

d    Set  the  HEATER  CAPT  selector  switch  to  HI .

e    Measure  the  blown  air  temperature  at  the  LH  foot

air-outlet .

NOTE  :  Measure  the  temperature  at  the  outlet .

NOTE  :  Make  sure  that  the  blown-air  stable  temperature

is  25  degrees  C  (77  degrees  F)  (+/-  7  degrees  C)

higher  than  the  temperature  shown  on  the AIR  COND

page  of  the  SD .

f g

Set  the  HEATER  F/O  selector  switch  to  LO .

Measure  the  blown  air  temperature  at  the  RH  foot

air-outlet .

NOTE  :  Measure  the  temperature  at  the  outlet .

NOTE  :  Make  sure  that  the  blown-air  stable  temperature

is  12 . 5  degrees  C  (54 . 5  degrees  F)  (+/-  7  degrees

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   73

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

C)  higher  than  the  temperature  shown  on  the  AIR COND  page  of  the  SD .

h    Set  the  HEATER  F/O  selector  switch  to  HI .

i    Measure  the  blown  air  temperature  at  the  right  foot

air-outlet .

NOTE  :  Measure  the  temperature  at  the  outlet .

NOTE  :  Make  sure  that  the  blown-air  stable  temperature

is  25  degrees  C  (77  degrees  F)  (+/-  7  degrees  C)

higher  than  the  temperature  shown  on  the AIR  COND

page  of  the  SD .

j    Switch  off  cabin  recirculation  fans  (pushbutton  on  the

30VU  panel)  and  keep  the  HEATER  CAPT  selector  switch  to

HI .

NOTE  :  Measure  the  temperature  at  the  outlet .

NOTE  :  Make  sure  that  the  blown-air  stable  temperature

is  the  same  as  the  value  shown  on  the  AIR  COND

page  of  the  SD  (+/-  7  degrees  C) 3    Close-up  :

a    On  the  AIR  COND  PNL  30VU  :

—  Release  the  PACK  1  and  the  PACK  2  pushbutton  switches

(OFF  legends  on) .

—  Set  the  COCKPIT  and  CABIN  temperature  selectors  to  the

12  o'clock  position .

—  Set  the  HEATER  CAPT  and  HEATER  F/O  selector  switches

to  OFF .

b    Stop  the  APU  (Refer  to  AMM  49-00-00  Page  block  201) .

c    Do  the  EIS  stop  procedure  (Refer  to  AMM  31-60-00  Page

block  201) .

d    De-energize  the  aircraft  electrical  circuits  (Refer  to

AMM  24-41-00  Page  block  201) .

## D .     CLOSE  UP

(1)    Config .  01

(a)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Multipurpose  Control  &  Display

Unit  (MCDU)  (3CA1,3CA2  and  3CA3)  (Refer  to  AMM  22-82-12  Page

block  401) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   74

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(b)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Radio  Management  Panel  (RMP)

(1RG1,  1RG2  and  1RG3)  (Refer  to  AMM  23-13-13  Page  block  401) .

(c)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Audio  Control  Panel  (ACP)  (2RN1,

2RN2,  2RN3,  2RN4  and  2RN5)  (Refer  to  AMM  23- 51-12  Page  block

401) .

(d)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the Air Traffic  Control/ Traffic Alert

and  Collision  Avoidance  System  (ATC/TCAS)  control  unit  (Refer

to  AMM  34- 52-12  Page  block  401) .

(e)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Weather  Radar  control  unit  (WXR)

(Refer  to  AMM  34-41-12  Page  block  401) .

(2)    Config .  02  thru  Config .  03

(a)    Make  sure  that  the  work  areas  are  clean  and  clear  of  tools  and

other  items  of  equipment .

(b)    Install  the  textile  floor  covering  (Refer  to AMM  25-13-44  Page

block  401) .

(c)    In  the  cockpit,  install  the  sidewall  and  ceiling  panels  211BW,

212BW,  211HC,  212FC,  212KC,  212JW  and  212HW  (Refer  to  AMM

06-41- 53  Page  block  001) .

(d)    Install  panel  Item  141  on  the  left  side  of  the  panel  120VU

(Refer  to  Figure  11    Sheet  1) .

(e)    Close  the  access  doors  811  and  822  (Refer  to  AMM  52-41-00  Page

block  001) .

(f)    Remove  the  access  platform(s) .

(g)    Restore  the  systems  and  the  aircraft  to  normal  operating

conditions .

## E .     DOCUMENTATION

Write  in  the  applicable  aircraft  records  that  you  have  done  all  the  work

given  in  the  Service  Bulletin .

## F .     EVALUATION

The  evaluation  period  must  be  done  for  six  months .

## G .     RESTORATION  OF  AIRCRAFT  TO  ITS  ORIGINAL  CONDITION

(1)    Config .  01    (Only  for  aircraft  MSN  2342)

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   75

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(a)    After  the  evaluation  period,  put  the  aircraft  back  to  the

initial  configuration .

1    Preparation  :

a    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Multipurpose  Control  &        Display  Unit  (MCDU)  (3CA1,3CA2  and  3CA3)  (Refer  to  AMM 22-82-12  Page  block  401) .

b    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Radio  Management  Panel  (RMP)

(1RG1,  1RG2  and  1RG3)  (Refer  to  AMM  23-13-13  Page  block

401) .

c    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Audio  Control  Panel  (ACP)     (2RN1,  2RN2,  2RN3,  2RN4  and  2RN5)  (Refer  to AMM  23- 51-12 Page  block  401) .

d    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the  Weather  Radar  control  unit

(WXR)  (3SQ)  (Refer  to  AMM  34-41-12  Page  block  401) .

e    Do  the  preparation  procedure  as  specified  in  the

removal/installation  of  the Air Traffic  Control/ Traffic

Alert  and  Collision  Avoidance  System  (ATC/TCAS)  control

unit  (3SH)  (Refer  to  AMM  34- 52-12  Page  block  401) .

f    Open,  safety  and  tag  these  circuit  breakers  :

DESIGNATION

PANEL

FIN

LOCATION

49VU

122VU

122VU

121VU

LIGHTING/EMER  LT/CKPT/DOME

LIGHTING/FLOOD/CTR  INST/PNL

LIGHTING/INSTL  LT/MAIN  INST/PNL

AND/PED

AIDS  &  RCDR/FDIMU

2    Modification  :

1LE

2LE

4LF

5TV

08  H

04  Z

04  Y

15  K

| a | Replace the panels B . (1) (a) . | modified at LH side in paragraph 3. |
| --- | --- | --- |
| b | Replace the panels B . (1) (b) . | modified at RH side in paragraph 3. |
| c | Replace the panels 3. B . (1) (c) . | assy modified at LH side in paragraph |
| d | Replace the panels 3. B . (1) (d) . | assy modified at RH side in paragraph |
| (2) Config . 02 |  |  |

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   76

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(a)    After  the  evaluation  period,  put  the  aircraft  back  to  the

initial  configuration .

1    Preparation  :

a    Make  sure  that  the  aircraft  is  electrically  grounded

(Refer  to  AMM  12- 34-24  Page  block  201) .

b    Put  the  access  platform(s)  in  position .

c    Open  the  access  doors  811  and  822  (Refer  to AMM  52-41-00

Page  block  001) .

d    In  the  cockpit,  remove  the  sidewall  and  ceiling  panels

211BW,  212BW,  211HC,  212FC,  212KC,  212JW  and  212HW

(Refer  to  AMM  06-41- 53  Page  block  001) .

e    Remove  the  panel  Item  (141)  on  the  left  of  panel  120VU

(Refer  to  Figure  11    Sheet  1)  for  access .

f    Remove  the  textile  floor  covering  (Refer  to AMM  25-13-44

Page  block  401) .

g    Make  sure  that  the  external  connector(s)  is  (are)  not

connected  to  the  aircraft  receptacle(s)  EXT  PW .

h    Make  sure  that  there  is  (are)  warning  notice(s)  on

external  power  receptacle  to  tell  persons  not  to  connect

an  external  power  source .

i    Make  sure  that  the  EMER  EXIT  LT  pushbutton  switch  on

25VU  is  on  OFF  position .

j    In  the  cocpkit,  on  the  overhead  panel

pushbutton  switches .

k    Put  a  warning  notice  on  refuel/defuel

tell  persons  no  to  operate  the  NORMAL

35VU  release  BAT

panel  800VU  to

BATTERY  switch .

l    Put  a  warning  notice  in  the  cockpit  to  tell  persons  not

to  start  the  Auxiliary  Power  Unit  (APU) .

m    Put  a  warning  notice  in  the  cockpit  to  tell  persons  not

to  start  the  ENG  1,  2.

n    Battery  disconnection  :

—  For  job  set-up,  refer  to  the  removal/installation  of

the  batteries  (Refer  to  AMM  24- 38- 51  Page  block  401) .

—  Turn  the  knurled  nut  a  quarter  turn .

—  Disconnect  the  connectors .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   77

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  Put  blanking  caps  on  the  disconnected  electrical

connectors .

2    Modification  :

a    In  the  cockpit,  remove  the  equipment  for  access  as

described  in  paragraph  3.  B .  (2)  (a) .

b    Remove  the  heater  3HE  at  LH  side  as  described  in

paragraph  3.  B .  (2)  (b) .

NOTE  :  After  the  removal  of  the  brackets,  install  rivets

ASNA2050DCJ3215    and  Rivet  ASNA2051DCJ2412    in

the  empty  holes .

NOTE  :  After  the  removal  of  the  grounding  points,

install  rivets  ASNA2050DCJ4828    in  the  empty

holes .

c    Remove  the  heater  4HE  at  RH  side  as  described  in

paragraph  3.  B .  (2)  (c) .

NOTE  :  After  the  removal  of  the  brackets,  install  rivets

ASNA2050DCJ3215    and  Rivet  ASNA2051DCJ2412    in

the  empty  holes .

NOTE  :  After  the  removal  of  the  grounding  points,

installrivets  ASNA2050DCJ4828    in  the  empty

holes .

d    Remove  the  circuit  breaker  and  the  wires  in  the  120VU

installed  in  paragraph  3.  B .  (2)  (d) .

e    Modify  the  equipment  and  remove  the  wires  between  the

cockpit  and  the  avionics  compartment  installed  in

paragraph  3.  B .  (2)  (e) .

f    In  the  RH  side,  remove  the  mounts  between  FR2  and  FR3

installed  in  paragraph  3.  B .  (2)  (f) .

g    In  the  LH  side,  remove  the  mounts  between  FR2  and  FR3

installed  in  paragraph  3.  B .  (2)  (g) .

h    In  the  avionics  compartment,  remove  the  module  installed

in  paragraph  3.  B .  (2)  (h) .

i    Remove  the  wires  and  connectors  between  the  equipment

(FINs  3HE  and  4HE)  and  the  avionics  compartment

installed  in  paragraph  3.  B .  (2)  (i) .

j    Remove  the  wire  installed  in  paragraph  3.  B .  (2)  (j) .

k    In  the  cockpit,  install  the  equipment  removed  for  access

as  described  in  paragraph  3.  B .  (2)  (k) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   78

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(3)    Config .  03

(a)    After  the  evaluation  period,  put  the  aircraft  back  to  the

initial  configuration .

1    Preparation  :

a    Make  sure  that  the  aircraft  is  electrically  grounded

(Refer  to  AMM  12- 34-24  Page  block  201) .

b    Put  the  access  platform(s)  in  position .

c    Open  the  access  doors  811  and  822  (Refer  to AMM  52-41-00

Page  block  001) .

d    In  the  cockpit,  remove  the  sidewall  and  ceiling  panels

211BW,  212BW,  211HC,  212FC,  212KC,  212JW  and  212HW

(Refer  to  AMM  06-41- 53  Page  block  001) .

e    Remove  the  panel  Item  (141)  on  the  left  of  panel  120VU

(Refer  to  Figure  11    Sheet  1)  for  access .

f    Remove  the  textile  floor  covering  (Refer  to AMM  25-13-44

Page  block  401) .

g    Make  sure  that  the  external  connector(s)  is  (are)  not

connected  to  the  aircraft  receptacle(s)  EXT  PW .

h    Make  sure  that  there  is  (are)  warning  notice(s)  on

external  power  receptacle  to  tell  persons  not  to  connect

an  external  power  source .

i    Make  sure  that  the  EMER  EXIT  LT  pushbutton  switch  on

25VU  is  on  OFF  position .

j    In  the  cocpkit,  on  the  overhead  panel

pushbutton  switches .

k    Put  a  warning  notice  on  refuel/defuel

tell  persons  no  to  operate  the  NORMAL

35VU  release  BAT

panel  800VU  to

BATTERY  switch .

l    Put  a  warning  notice  in  the  cockpit  to  tell  persons  not

to  start  the  Auxiliary  Power  Unit  (APU) .

m    Put  a  warning  notice  in  the  cockpit  to  tell  persons  not

to  start  the  ENG  1,  2.

n    Battery  disconnection  :

—  For  job  set-up,  refer  to  the  removal/installation  of

the  batteries  (Refer  to  AMM  24- 38- 51  Page  block  401) .

—  Turn  the  knurled  nut  a  quarter  turn .

—  Disconnect  the  connectors .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   79

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  Put  blanking  caps  on  the  disconnected  electrical

connectors .

2    Modification  :

a    In  the  cockpit,  remove  the  equipment  for  access  as

described  in  paragraph  3.  B .  (3)  (a) .

b    Remove  the  heater  3HE  at  LH  side  as  described  in

paragraph  3.  B .  (3)  (b) .

NOTE  :  After  the  removal  of  the  brackets,  install  rivets

ASNA2050DCJ3215    and  Rivet  ASNA2051DCJ2412    in

the  empty  holes .

NOTE  :  After  the  removal  of  the  grounding  points,

install  rivets  ASNA2050DCJ4828    in  the  empty

holes .

c    Remove  the  heater  4HE  at  RH  side  as  described  in

paragraph  3.  B .  (3)  (c) .

NOTE  :  After  the  removal  of  the  brackets,  install  rivets

ASNA2050DCJ3215    and  Rivet  ASNA2051DCJ2412    in

the  empty  holes .

NOTE  :  After  the  removal  of  the  grounding  points,

installrivets  ASNA2050DCJ4828    in  the  empty

holes .

d    Remove  the  circuit  breaker  and  the  wires  in  the  120VU

installed  in  paragraph  3.  B .  (3)  (d) .

e    Modify  the  equipment  and  remove  the  wires  between  the

cockpit  and  the  avionics  compartment  installed  in

paragraph  3.  B .  (3)  (e) .

f    In  the  RH  side,  remove  the  mounts  between  FR2  and  FR3

installed  in  paragraph  3.  B .  (3)  (f) .

g    In  the  LH  side,  remove  the  mounts  between  FR2  and  FR3

installed  in  paragraph  3.  B .  (3)  (g) .

h    In  the  avionics  compartment,  remove  the  module  installed

in  paragraph  3.  B .  (3)  (h) .

i    Remove  the  wires  and  connectors  between  the  equipment

(FINs  3HE  and  4HE)  and  the  avionics  compartment

installed  in  paragraph  3.  B .  (3)  (i) .

j    Remove  the  wire  installed  in  paragraph  3.  B .  (3)  (j) .

k    In  the  cockpit,  install  the  equipment  removed  for  access

as  described  in  paragraph  3.  B .  (3)  (k) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   80

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

## H .    TEST

(1)

PANEL

Config .  01

(a)    Remove  the

breakers  :

DESIGNATION

safety  clips  and  tags  and  close  these  circuit

FIN

LOCATION

49VU

122VU

122VU

121VU

1LE

2LE

4LF

5TV

08  H

04  Z

04  Y

15  K

LIGHTING/EMER  LT/CKPT/DOME

LIGHTING/FLOOD/CTR  INST/PNL

LIGHTING/INSTL  LT/MAIN  INST/PNL

AND/PED

AIDS  &  RCDR/FDIMU

(b)    Do  the  tests  after  removal/installation  of  the  MCDU  (3CA1

,3CA2  and  3CA3)  (Refer  to  AMM  22-82-12  Page  block  401) .

(c)    Do  the  tests  after  removal/installation  of  the  RMP  (1RG1,  1RG2

and  1RG3)  (Refer  to  AMM  23-13-13  Page  block  401) .

(d)    Do  the  tests  after  removal/installation  of  the  ACP  (2RN1,

2RN2,  2RN3,  2RN4  and  2RN5)  (Refer  to  AMM  23- 51-12  Page  block

401) .

(e)    Do  the  tests  after  removal/installation  of  the  Weather  Radar

control  unit  (3SQ)  (Refer  to  AMM  34-41-12  Page  block  401) .

(f)    Do  the  tests  after  removal/installation  of  the  ATC/TCAS

control  unit  (3SH)  (Refer  to  AMM  34- 52-12  Page  block  401) .

(g)    For  the  panel  111VU,  do  an  operational  test  of  the  general

illumination  (Refer  to  AMM  33-12-00  Page  block  501),

(h)    Do  an  operational  test  of  the  instrument  and  panel  integral

lighting  (Refer  to  AMM  33-13-00  Page  block  501)

(i)    Do  an  operational  test  of  the  Cockpit  Door  Lock  System  (CDLS)

(Refer  to  AMM  52- 51-00  Page  block  501) .

(2)    Config .  02  thru  Config .  03

(a)    Job  set-up  :

1    Re-connect  the  battery  :

—  For  job  set-up,  refer  to  the  removal/installation  of  the

batteries  (Refer  to  AMM  24- 38- 51  Page  block  401) .

—  Remove  the  blanking  cap  from  the  electrical  connectors .

—  Make  sure  that  the  electrical  connectors  are  clean  and  in

correct  condition .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   81

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

—  Connect  the  connector  of  the  batteries .

—  Turn  the  knurled  nut  a  quarter  turn .

2    Remove  all  the  warning  notices .

3    Restore  the  systems  and  the  aircraft  to  the  normal

operating  condition .

(b)    Preliminary  test

1    Initial  conditions  :

a    On  each  connectors  that  you  disconnected,  do  a  visual

check  as  defined  by  ESPM  20- 52-10,  to  make  sure  that  :

—  The  label  of  the  plug  is  the  same  as  the  label  on  the

receptacle .

—  The  plug  is  correctly  locked .

2    Preliminary  test  :

NOTE  :  If  the  aircraft  is  operated  in  CAT  3  conditions,  you

must  also  do  this  test  :  land  CAT  3  capability  test

(Refer  to  AMM  22-97-00  Page  block  501) .

a    Do  an  operational  test  of  the  instrument  and  panel

integral  lighting  (Refer  to  AMM  33-13-00  Page  block

501) .

b    Do  an  operational  test  of  the  annunciator  light  test

system  in  the  cockpit  (Refer  to  AMM  33-14-00  Page  block

501) .

c    Do  an  operational  test  of  the  avionics  equipment

ventilation  system  (Refer  to  AMM  21-26-00  Page  block

501) .

d    Do  an  operational  test  of  the  Automatic  Flight  System

(AFS)  (Refer  to  AMM  22-96-00  Page  block  501) .

e    Do  an  operational  test  of  the  Electronic  Instrument

System  (EIS)  (Refer  to  AMM  31-60-00  Page  block  501) .

f    Do  an  operational  test  of  the  Electronic  Centralized

Aircraft  Monitoring  (ECAM)  (Refer  to  AMM  31- 50-00  Page

block  501) .

g    Do  the  ground  scanning  of  the  central  warning  system

(Refer  to  AMM  31- 50-00  Page  block  501) .

(c)    Tests  after  disconnection/connection  of  the  plug  1180VC  :

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   82

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

1    Do  an  operational  test  of  the  Ground  Power  Control  Unit

(GPCU)  with  the  Centralized  Fault  Display  System  (CFDS)

(Refer  to  AMM  24-41-00  Page  block  501) .

(d)    Tests  after  disconnection/connection  of  the  plug  1025VC  :

1    Do  an  operational  test  of  the  Cockpit  Voice  Recorder  (CVR)

and  of  the  CVR  channel  recording  (Refer  to  AMM  23-71-00

Page  block  501) .

(e)    Tests  after  disconnection/connection  of  the  plug  1200VC  :

1    Do  an  operational  test  of  the  parking  brake  system  (Refer

to  AMM  32-45-00  Page  block  501) .

(f)    Tests  after  disconnection/connection  of  the  plug  2252VC  :

1    Do  an  operational  test  of  the  engine  fire  and  overheat

detection  (Refer  to  AMM  26-12-00  Page  block  501) .

(g)    Test  after  removal/installation  of  the  equipment  :

1    Do  the  test  after  removal/installation  of  the  captain  and

first  officer  seats  (Refer  to  AMM  25-11- 51  Page  block  401) .

2    Do  the  test  after  removal/installation  of  the  captain  and

first  officer  pedals  (Refer  to  AMM  27-21-42  Page  block

401) .

3    Do  the  test  after  removal/installation  of  the  captain  and

first  officer  foot warmer  (Refer  to AMM  25-15- 51  Page  block

401) .

(h)    Test  after  removal/installation  of  the  panels  :

1    Do  the  test  after  removal/installation  of  the  panel  35VU

(Refer  to  AMM  31-10-00  Page  block  201) .

2    Do  the  test  after  removal/installation  of  the  panel  40VU

(Refer  to  AMM  31-10-00  Page  block  201) .

3    Do  the  test  after  removal/installation  of  the  panel  48VU

(Refer  to  AMM  31-10-00  Page  block  201) .

4    Do  the  test  after  removal/installation  of  the  ENG/APU  fire

panel  (Refer  to  AMM  26-12-12  Page  block  401) .

(i)    Test  after  removal/installation  of  the  30VU  panel  :

1    Do  an  operational  test  of  the  flow  control  and  indicating

system  (Refer  to  AMM  21- 51-00  Page  block  501) .

2    Do  an  operational  test  of  the  flow  control  valves  closing

(Refer  to  AMM  21- 51-00  Page  block  501) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   83

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

3    Do  an  operational  test  of  the  ram-air  inlet  (Refer  to  AMM

21- 55-00  Page  block  501) .

4    Do  a  functional  test  of  the  cockpit  and  cabin  temperature

control  without  Centralized  Fault  Display  System  (CFDS)  /  Multipurpose  Control  and  Display  Unit  (MCDU)  (Refer  to  AMM 21-63-00  Page  block  501) .

5    Do  an  operational  test  of  the  bleed  air  system  (Refer  to

AMM  36-11-00  Page  block  501) .

(j)    Do  a  voltage  check  at  pin  2  of  each  following  circuit  breakers

to  make  sure  that  there  is  115VAC  :

—  122VU  :  1LC2 .

(k)    Do  an  operational  test  of  the  Circuit  Breaker  Monitoring

System  (CBMS)  (Refer  to  AMM  31- 50-00  Page  block  501) .

## I .     CLOSE  UP

(1)    Config .  01

(a)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Multipurpose  Control  &  Display

Unit  (MCDU)  (3CA1,3CA2  and  3CA3)  (Refer  to  AMM  22-82-12  Page

block  401) .

(b)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Radio  Management  Panel  (RMP)

(1RG1,  1RG2  and  1RG3)  (Refer  to  AMM  23-13-13  Page  block  401) .

(c)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Audio  Control  Panel  (ACP)  (2RN1,

2RN2,  2RN3,  2RN4  and  2RN5)  (Refer  to  AMM  23- 51-12  Page  block

401) .

(d)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the Air Traffic  Control/ Traffic Alert

and  Collision  Avoidance  System  (ATC/TCAS)  control  unit  (Refer

to  AMM  34- 52-12  Page  block  401) .

(e)    Do  the  close-up  procedure  as  specified  in  the

removal/installation  of  the  Weather  Radar  control  unit  (WXR)

(Refer  to  AMM  34-41-12  Page  block  401) .

(2)    Config .  02  thru  Config .  03

(a)    Make  sure  that  the  work  areas  are  clean  and  clear  of  tools  and

other  items  of  equipment .

(b)    Install  the  textile  floor  covering  (Refer  to AMM  25-13-44  Page

block  401) .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   84

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

(c)    In  the  cockpit,  install  the  sidewall  and  ceiling  panels  211BW,

212BW,  211HC,  212FC,  212KC,  212JW  and  212HW  (Refer  to  AMM

06-41- 53  Page  block  001) .

(d)    Install  the  panel  Item  (141)  on  the  left  of  the  panel  120VU

(Refer  to  Figure  11    Sheet  1)  for  access .

(e)    Close  the  access  doors  811  and  822  (Refer  to  AMM  52-41-00  Page

block  001) .

(f)    Remove  the  access  platform(s) .

(g)    Restore  the  systems  and  the  aircraft  to  normal  operating

conditions .

## J .     DOCUMENTATION

Write  in  the  applicable  aircraft  records  that  you  have  done  all  the  work

given  in  the  Service  Bulletin .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   85

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  1  Sheet  1

Config .  01  thru  03  :  Removal  of  the  Duct  for  Access .

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   86

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  2  Sheet  1

Config .  02  thru  03  :  Installation  of  the  Seal

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   87

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  2  Sheet  2

Config .  02  thru  03  :  Installation  of  the  Seal

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   88

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  3  Sheet  1

Config .  01  :  Replacement  of  the  Pylon  Panels

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   89

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  3  Sheet  2

Config .  01  :  Replacement  of  the  Pylon  Panels

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   90

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  4  Sheet  1

Config .  02  thru  03  :  Installation  of  the  Seal  on  the  Panels  Assy

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   91

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  4  Sheet  2

Config .  02  thru  03  :  Installation  of  the  Seal  on  the  Panels  Assy

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   92

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  5  Sheet  1

Config .  02  thru  03  :  Replacement  of  the  Cabin  Floors

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   93

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  6  Sheet  1

Config .  02  thru  03  :  Installation  of  the  Ducts  and  the  Heaters  3HE  and  4HE

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   94

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  6  Sheet  2(Goto  A3  section)

Config .  02  thru  03  :  Installation  of  the  Ducts  and  the  Heaters  3HE  and  4HE

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   95

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  6  Sheet  3

Config .  02  thru  03  :  Installation  of  the  Ducts  and  the  Heaters  3HE  and  4HE

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   97

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  6  Sheet  4

Config .  02  thru  03  :  Installation  of  the  Ducts  and  the  Heaters  3HE  and  4HE

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   98

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  6  Sheet  5

Config .  02  thru  03  :  Installation  of  the  Ducts  and  the  Heaters  3HE  and  4HE

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   99

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  7  Sheet  1

Config .  02  thru  03  :  Installation  of  the  Brackets

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   100

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  7  Sheet  2

Config .  02  thru  03  :  Installation  of  the  Brackets

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   101

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  7  Sheet  3

Config .  02  thru  03  :  Installation  of  the  Brackets

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   102

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  7  Sheet  4

Config .  02  thru  03  :  Installation  of  the  Brackets

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   103

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  8  Sheet  1

Config .  02  thru  03  :  Installation  of  the  Ground  Points  and  of  their  Placards  at

LH  side

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   104

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  8  Sheet  2

Config .  02  thru  03  :  Installation  of  the  Ground  Points  and  of  their  Placards  at

LH  side

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   105

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  9  Sheet  1

Config .  02  thru  03  :  Installation  of  the  Ground  Points  and  of  their  Placards  at

RH  side

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   106

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  9  Sheet  2

Config .  02  thru  03  :  Installation  of  the  Ground  Points  and  of  their  Placards  at

RH  side

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   107

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  10  Sheet  1

Config .  02  thru  03  :  Location  of  the  Work  Area

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   108

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  11  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Equipment  in  the  Rear  C/B  Panel  122VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   109

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  11  Sheet  2

Config .  02  thru  03  :  Modification  of  the  Equipment  in  the  Rear  C/B  Panel  122VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   110

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  12  Sheet  1

Config .  02  thru  03  :  Location  of  the  Equipment  for  Access

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   111

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  13  Sheet  1

Config .  02  :  Modification  of  the  Equipment  in  the  Overhead  Panel  20VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   112

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  13  Sheet  2

Config .  02  :  Modification  of  the  Equipment  in  the  Overhead  Panel  20VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   113

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  14  Sheet  1

Config .  03  :  Modification  of  the  Equipment  in  the  Overhead  Panel  20VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   114

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  14  Sheet  2

Config .  03  :  Modification  of  the  Equipment  in  the  Overhead  Panel  20VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   115

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  15  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Routing  in  the  RH  Side  between  FR2  and

FR3

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   116

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  15  Sheet  2

Config .  02  thru  03  :  Modification  of  the  Routing  in  the  RH  Side  between  FR2  and

FR3

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   117

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  16  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Routing  in  the  LH  Side  between  FR2  and

FR3

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   118

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  16  Sheet  2

Config .  02  thru  03  :  Modification  of  the  Routing  in  the  LH  Side  between  FR2  and

FR3

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   119

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  17  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Equipment  in  the  Avionics  Compartment

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   120

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  18  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Wiring  in  the  Rear  Panel  120VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   121

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  19  Sheet  1

Config .  02  thru  03  :  30VU  Wiring  Diagram  -  For  Information  Only

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   122

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  20  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Wiring  between  the  Cockpit  and  the

Avionics  Compartment

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   123

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  21  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Wiring  between  the  Equipment  and  the

Avionics  Compartment

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   124

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  22  Sheet  1

Config .  02  thru  03  :  Modification  of  the  Wiring  in  the  Relay  Box  103VU

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   125

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

This  Page  Intentionally  Left  Blank

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   126

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  23  Sheet  1(Goto  A3  section)

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   127

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  24  Sheet  1(Goto  A3  section)

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   129

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  25  Sheet  1(Goto  A3  section)

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   131

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  26  Sheet  1(Goto  A3  section)

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06                                      SERVICE  BULLETIN  No .  :  A320— 21— 1908

REVISION  No .  :  01  —  Mar  30/07                                            Page  :   133

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

# REPORTING  SHEET

TITLE  :  AIR  CONDITIONING  -  COCKPIT  AIR  HEATING  -  IMPROVE  FOOT  WARMER  EFFICIENCY .

MODIFICATION  No .  :  35861P9435  35861P9449

Please  complete  the  appropriate  item  (A  or  B):

A  -  SB  WILL  BE  embodied   .   .   .   .   .   .   .   .   .   .   .   .   .  YES/NO  (if  NO  please  comment)

If  YES,  aircraft  concerned   (as  per  SB  effectivity  by  default)  and  planned  dates

(month/year)  of  embodiment :   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

B  -  SB  HAS  BEEN  embodied  on  aircraft :  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

Operator  comments :  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

.  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .

From  Airline :   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .

Name/Title :   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .

Signature :   .   .   .   .   .   .   .   .   .   .   .   .   .   .                      Date :   .   .   .   .   .   .   .   .   .   .   .   .   .

If  operational  documentation  is  affected  (see  Paragraph  1 .K  of  this  SB):  If      information  is  needed  prior  to  next  normal  revision  or  prior  to  SB  embodiment, please  indicate  required  service:

Either:  Modification  Operational  Impact   (MOI),  if  available .   .  YES/NO

Or         :  Intermediate  revision .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  YES/NO

Important  Information: This  SB  will  only  be  incorporated  in  your  maintenance  and

operational  documentation  if  this  sheet  is   returned  to  Airbus  and  signed  by  a

duly  authorised   representative .  With  the  next  feasible   revision,  this  will

result  in

-  updating  of  maintenance  documentation  to  show  pre  and  post  SB  data .

-  updating  of  maintenance  and  operational  documentation  to  show  post  SB  data

after  embodiment .

If  this  SB  requires  previous  or  simultaneous  accomplishment  of  other  SBs,

Airbus  shall  automatically  include  them  in  the  manual   revisions .  Refer  to  SIL

00-037  for  detailed  information .

Please  return  this  completed  sheet  to:

AIRBUS

CUSTOMER  SERVICES  DIRECTORATE

1  Rond  Point  Maurice  Bellonte

31707  BLAGNAC  CEDEX  FRANCE

Attn:  SDC32  Technical  Data  and  Documentation  Services

FAX:  (+33)  5  61  93  28  06

e -mail:  sb . reporting@airbus .com

or  via  your  Resident  Customer  Support  Office .

Alternatively,  SB  lists  via  letters  or  fax  are  also  accepted .

5    DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  1  of  1

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

# QUALITY  PERCEPTION  FORM

Use  this  form  to  tell  us  what  is  your  perception  of  the  quality  of  this  Service Bulletin . The  reported  data  that  you  provide  us will  be  used  to  analyse  areas  of difficulties  and  to  take  corrective  action  to  further  improve  the  quality  of  our Service  Bulletins .

We  thank  you  for  the  time  you  have  taken  in  completing  this  form .

(Please  rate  on  a  scale  of  1  to  4,  with  4  being  the  highest  score)

| - Quality rating of this SB | 4 3 | 2 | 1 |
| --- | --- | --- | --- |
| - Quality rating of the Accomplishment Instructions | 4 3 | 2 | 1 |
| - Quality rating of the Illustrations | 4 3 | 2 | 1 |
| - Is this SB easy to understand ? | Y / N |  |  |

If  you  have  had  difficulties  in  the  accomplishment  of  this  SB  please  quote  below

the  area(s)  and  give  a  short  description  of  the  issue .

Planning

X    Effectivity

X    Reason

X    Manpower

X    References

X    Publication

Comments  :

Material

X   Kit  content

X   List  of  Materials

Operator  Supplied

X   Re-identification

X  Tooling

Instructions

X   Preparation

X  Mod/Inspection

X  Test

X  Close-Up

X   Illustrations

Date:

Operator  :

Name/Title  :

Please  return  this  form  to  :

AIRBUS

CUSTOMER  SERVICES  DIRECTORATE

1  Rond  Point  Maurice  Bellonte

31707  BLAGNAC

Attn  :  SEM4    Service  Bulletins  Management

FAX  :  (33)  5  61  93  42  51

Or  via  your  Resident  Customer  Support  Office .

5    DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  1  of  1

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

# SERVICE  BULLETIN

Figure  6  Sheet  2

Config .  02  thru  03  :  Installation  of  the  Ducts  and  the  Heaters  3HE  and  4HE

DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  95/96

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  23  Sheet  1

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  127/128

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  24  Sheet  1

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  129/130

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  25  Sheet  1

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  131/132

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved

@ Ac18/Ac16/Ac20/Ac21

SERVICE  BULLETIN

Figure  26  Sheet  1

Config .  02  thru  03  :  Hook-up  Chart

DATE  :  Mar  08/06

REVISION  No .  :  01  —  Mar  30/07

SERVICE  BULLETIN  No .  :  A320— 21— 1908

Page  :  133/134

Printed  in  FRANCE  ©  AIRBUS,  2007  All  rights  reserved